LIMITED DISTRIBUTION

WORKING MATERIAL

Draft

Working Document, Version 1.4, 31 May 2001

GUIDANCE ON RADIOLOGICAL ASSESSMENT PROCEDURES for the protection of human health TO DETERMINE IF MATERIALS FOR DISPOSAL AT SEA ARE WITHIN THE SCOPE OF THE LONDON CONVENTION 1972

INTERNATIONAL ATOMIC ENERGY AGENCY

WAGRAMER STRASSE 5, P.O. BOX 100, A-1400 VIENNA, AUSTRIA

TELEPHONE: (+43 1) 2600, FACSIMILE: (+43 1) 26007, TELEX: 112645 ATO, E-MAIL: Official.Mail@iaea.org, INTERNET: http://www.iaea.org

CONTENTS

1. INTRODUCTION

2. GUIDANCE FOR DETERMINING WHETHER CANIDATE MATERIALS ARE DE MINIMIS IN THE CONTEXT OF THE LONDON CONVENTION

2.1. Stepwise Evaluation Procedure

2.2. Objectives and Requirements of a Specific Assessment

2.3. Environmental Protection

2.4. Application of the Waste Assessment Guidance

3. THE SPECIFIC ASSESSMENT PROCESS

3.1. Introduction

3.2. De Minimis Decision Criteria

3.3. Data Collection

3.4. Screening Assessment

3.4.1. Considerations

3.4.2. Exposure pathways

3.4.3. Procedure for Calculating Individual Dose

3.4.4. Procedure for Calculating Collective Dose

3.4.5. Comparison with Decision Criteria

3.5. Detailed Specific Assessment

REFERENCES

APPENDIX I PROCEDURE FOR ASSESSING INDIVIDUAL AND COLLECTIVE DOSES ARISING FROM DISPOSALS IN COASTAL SEAS

REFERENCES

APPENDIX II PARAMETERS INCLUDING DEFAULT VALUES AND SUGGESTED INFORMATION SOURCES

REFERENCES

ANNEX I (FROM THE CONTRACTING PARTIES TO THE LC) GUIDELINES FOR THE APPLICATION OF THE DE MINIMIS CONCEPT UNDER THE LONDON CONVENTION1

ANNEX II EXAMPLE CALCULATIONS

ANNEX III INFORMATION ON CAUSES/SOURCES AND LABORATORY RADIOMETRIC PROCEDURES

CONTRIBUTORS TO DRAFTING AND REVIEW

# introduction

The Convention on the Prevention of Marine Pollution by Dumping of Wastes and Other Matter (London Convention 1972) prohibits the disposal at sea of radioactive wastes and other radioactive matter. However, all materials, including natural and inert materials, contain natural radionuclides and are frequently contaminated with artificial radionuclides from such anthropogenic sources as fallout due to past atmospheric nuclear testing. Therefore, the Contracting Parties to the London Convention recognized the need to develop definitions and guidelines whereby candidate materials (those wastes or other matter not otherwise prohibited from disposal at sea in accordance with Annex I to the Convention) containing less than de minimis levels of specific activity, below which material would not be regarded as ‘radioactive’, could be disposed of pursuant to the provisions of the Convention.

At the 19th Consultative Meeting in 1997, Contracting Parties agreed to request the IAEA to develop further the concept of de minimis and, in particular, to “provide guidance for making judgements on whether materials planned to be dumped could be exempted from radiological control or whether a specific assessment was needed” (LC 19/10, paragraph 6.31) [1]. This paragraph continues: “The IAEA would then further be requested to provide guidance to national authorities responsible for conducting specific assessments.”

In 1999, the IAEA presented its advice on de minimis in IAEA-TECDOC-1068 [2], entitled “Application of Radiological Exclusion and Exemption Principles to Sea Disposal” to the 20th Consultative Meeting of Contracting Parties. The Contracting Parties to the London Convention accepted these principles and criteria at their 21st Consultative Meeting in October 1999 and interpreted them further in the Guidelines for the Application of the de minimis concept under the London Convention (the Guidelines) (LC 21/13, Annex 6) [3]. At that time the Contracting Parties asked the IAEA to prepare additional guidelines on the conduct of specific assessments to determine whether candidate materials for sea disposal contained de minimis levels of radioactivity.

This Report contains guidance on performing specific radiological assessments of disposals of candidate materials to determine whether the materials are de minimis in the meaning of the London Convention.

Section 2 provides a summary of the Step Evaluation Procedure as detailed in the Guidelines for the Application of the de minimis concept under the London Convention and background information needed to understand the context of this guidance. Section 3 describes in detail the specific assessment process of the radioactive properties of a candidate material for disposal. It contains a schematic diagram illustrating the specific assessment process and calculation factors that can be used in the screening calculation of the dose from a given activity concentration in a candidate material. Additional specific guidance for characterising the radiation properties of the candidate material and assessing their impact is provided in a series of Appendices and Annexes.

Appendix I describes an example procedure which radiological protection authorities may find useful when planning and performing more detailed assessments of doses arising from disposal of a candidate material. Appendix II provides a detailed description of the parameters which form part of the procedure described in Appendix I and an explanation of their function in that model and suggests generic values for these parameters.

Annex I contains the Guidelines for the Application of the de minimis concept under the London Convention in full. Annex II provides a number of practical example calculations using the dose calculation factors provided in Section 3. Annex III gives indications of the type of information, and information sources, that might be necessary to complete the specific assessment process presented as a series of questions. It also provides simple guidance on monitoring techniques appropriate for various sample types and radionuclides.

# guidance for determining whether canidate materials are
de minimis in the context of the london convention

## Stepwise Evaluation Procedure

A summary of the Stepwise Evaluation Procedure adopted by the 21st Consultative Meeting of the Contracting Parties to the London Convention is given in Box 1 below. The complete procedure is provided in Annex I.

| Box 1: Stepwise Evaluation Procedure |  |
| --- | --- |
| Step 1: Candidate material |  |
|  | - Are the proposed materials eligible for dumping under the provisions of the London Convention? - If no, the material is not allowed to be dumped and no further consideration is warranted. - If yes, go to Step 2. |
| - Step 2: Initial screen for sources of contamination |  |
|  | - Is there reason to believe that the candidate material contains anything other than unmodified natural radionuclides at background comparable with that in the receiving environment and artificial radionuclides derived from global fallout? - If no, the materials are de minimis. - If yes, go to Step 3. |
| - Step 3: Assessment of additional causes/sources |  |
|  | - What are the likely additional causes/sources contributing to the radioactivity in the materials? - If only unmodified natural causes/sources, go to Step 4. - If only anthropogenic causes/sources, go to Step 5. - If both anthropogenic and natural causes/sources, go to Step 5. |
| - Step 4: Natural causes/sources |  |
|  | - If the material were to be dumped, would it substantially increase radioactivity at the dump site? - If no, the materials are de minimis. - If yes, go to Step 6. |
| - Step 5: Anthropogenic causes/sources |  |
|  | - Were the likely anthropogenic causes/sources part of exempted or cleared practices or excluded exposures? - If no, go to Step 6 - If yes, were the marine environmental exposure pathways considered by the national radiation protection authority and are these suitable to an assessment of the proposed dumping operation? - If yes, the materials are de minimis. - If no, go to Step 6. |
| - Step 6: Specific assessment |  |
|  | Materials not determined to be de minimis through the evaluation in Steps 1–5 of Annex I may be determined to be de minimis by the application of a specific assessment. |

There are various reasons for which a candidate material may require a specific assessment as a result of reaching Step 6 of the Stepwise Evaluation Procedure. These are outlined below, not necessarily in the order of radiological significance:

- When the national permitting authority is concerned that the radioactivity would be substantially increased at the dumping site as a result of the dumping of candidate materials of natural origin unmodified by human activities;

- When the likely anthropogenic causes/sources of artificial radionuclides and/or altered natural radionuclides in the candidate material were not part of exempted or cleared practices or excluded exposures;

- When, despite the likely anthropogenic causes/sources of artificial radionuclides and/or altered natural radionuclides in the candidate material being part of exempted or cleared practices or excluded exposures, the marine environmental exposure pathways were either:

- not considered by the national radiation protection authority; or

- not suitable to an assessment of the proposed dumping operation.

## Objectives and Requirements of a Specific Assessment

The objective of a specific assessment is to determine if the candidate materials can be assigned as de minimis in accordance with IAEA-TECDOC-1068. A specific assessment should provide an evaluation of the potential adverse radiological impacts to the marine environment including effects upon human health and to flora and fauna, and to other legitimate uses of the sea. International radiological exemption criteria, as provided in the International Basic Safety Standards and used in IAEA-TECDOC-1068, are based on the protection of human health and are the only currently available basis for the assessments contained in this Report.

The nature and extent of the specific assessment should be determined in accordance with existing knowledge of the sources and likely extent of any radioactive contamination of the candidate material. For example, candidate materials comprising sediments containing only relatively minor amounts of radionuclides may not need to be subjected to an unnecessarily detailed or unnecessarily complex assessment process.

## Environmental Protection

Currently there are no criteria which would enable a specific assessment of the effects on flora and fauna to be evaluated. The use of human protection criteria and practices to infer protection of the environment from effects of ionizing radiation has been supported by statements by the International Commission on Radiological Protection, ICRP [4]. However, it is recognized, for example in IAEA TECDOC-1091 [5], that reliance on human radiological protection criteria may not be adequate for all possible space and time scales. The ICRP approach is used in many countries for protection of the environment, but several countries have recognized the need to develop guidance and criteria to explicitly demonstrate that the environment is protected. 	There is, as yet, no clear international consensus on the guidelines, endpoints or targets that may be used as a basis for environmental protection.

A number of relevant multilateral commitments have been made to protect the environment from the effects of ionizing radiation and various approaches are already being developed and adopted in some countries. Work is in progress at the IAEA, the European Union, the International Union of Radioecology, and in a number of Member States, to develop approaches and criteria for the protection of the environment from the effects of ionizing radiation. One of the objectives of these developments is to reduce uncertainties by improved knowledge, and thus achieve a greater confidence that criteria will provide the desired level of environmental protection.

The Guidelines for the Application of the de minimis Concept under the London Convention (LC 21/13, Annex 6) [3], state that, “... until complementary international radiological criteria for the protection of flora and fauna are developed, permitting authorities are advised to use appropriate scientific information and a precautionary approach (as provided for in resolution LDC 44(14)) [6] in conducting assessments of the potential impacts on the marine environment” (Step 6 of the Step Evaluation Procedure). Further, it is stated that “The field of radiological protection is evolving. Contracting Parties should take all relevant advances into account when applying the guidelines. For example, criteria for evaluating the impacts of radioactivity on the marine environment are advancing, and should, when available and as relevant, be expeditiously utilised.” (Section 4.3).

## Application of the Waste Assessment Guidance

Candidate materials that are determined to be de minimis through the evaluation in Steps 1–6 above must also be evaluated through application of the generic Guidelines for the Assessment of Wastes or Other Matter that May be Considered for Dumping, and/or the waste-specific guidance developed thereunder. No further evaluation of the radionuclide content of the candidate materials that are de minimis is needed.

# The Specific Assessment PROCESS

## Introduction

This Section describes a process for carrying out a specific assessment for the protection of human health to determine whether a candidate material is de minimis under the London Convention (Step 6 of the Step Evaluation Procedure, see Box 1).

The specific assessment is exclusively designed to calculate radiation doses received by human beings who could be exposed to the radioactivity in the candidate material. The evaluation of the radiological impact on the environment from the disposal at sea of a candidate material is not included in this process.

The schematic diagram in Figure 1 outlines the process for performing a specific assessment including guidance on gathering information. As indicated in the figure, there are two levels of assessment contained in this process: a screening assessment and, if necessary, a more detailed assessment. The first assessment is based on conservative dose per unit activity concentration factors, which are provided in this Section. The second assessment is more detailed and requires that additional information be collected and input from radiation protection and modelling specialists will be needed. A suggested process for carrying out a detailed assessment, including the associated parameters, is described in the Appendices I and II to this Report.

## De Minimis Decision Criteria

A specific assessment for the protection of human health should include calculations of individual doses as well as collective doses for comparison with criteria for exemption for the purposes of the London Convention as given in IAEA-TECDOC-1068 and summarized in the concluding section of that Report as follows:

"A practice, or source within a practice, may be exempted without further consideration provided that the following radiological criteria are met in all feasible situations:

(a)	the effective dose expected to be incurred by any member of the public due to the exempted practice or source is of the order of 10 mSv or less in a year; and

(b)	either the collective effective dose committed by one year of performance of the practice is not more than about 1 man.Sv or an assessment for the optimization of protection shows that exemption is the optimum option."

## Data Collection

Information necessary for the screening assessment process includes estimates of the quantity of the candidate materials together with its content and activity concentrations of the radionuclides present in the material. Knowledge of the causes and sources can be useful in determining which radionuclides are likely to be present in the candidate material. Such sources could include, but are not limited to, nuclear medicine facilities, nuclear power plants, nuclear processing facilities, mining activities and fossil fuel power plants. Mining could include the mining and processing of phosphate, tin, niobium, thorium, titanium, natural gas, uranium and precious metals. For additional information regarding the identification of potential causes or sources of radionuclides see Table III-II and Table III-III in Annex III.

FIG. 1. Step 6 of the Stepwise Evaluation Procedure: The specific assessment process.

Based on the identified causes and sources, data should be obtained from the radiation protection authority regarding the known activity concentrations and release of radionuclides as a result of these sources. It is not expected that new measurements will be routinely needed for the screening process as sufficient information should already be available. If necessary, analysis of radionuclide content and activity of the candidate material should be conducted. For additional information regarding the analysis of radionuclides see Annex II.

## Screening Assessment

This Section contains a screening procedure for assessing whether a candidate material is de minimis under the London Convention. This approach uses dose per unit activity concentration factors which have been derived on the basis of conservative assumptions and the procedure outlined in Appendices I and II. These factors allow individual and collective doses to be calculated from activity concentrations of radionuclides in the candidate materials. The activity concentrations of these radionuclides must be obtained for the material in question and for each relevant radionuclide and should be representative of the material in question and provide sufficient averaging over volume and time.

Dose per unit activity concentration factors provided in Tables 3.1 and 3.2 take account of the build-up of the progeny in the candidate material and in other environmental materials as explained in Section II.2 of Appendix II. Two values are provided for U-238 in the tables. The value for U-238 should be used when activity concentrations of Ra-226 are available. If these activity concentrations are not available, the values for U-238s should be used instead.

Two groups of individuals are considered who may receive the highest doses from radionuclides present in material proposed to be disposed of at sea. The first group consists of individuals working on the ship, the crew, who transport the material from its point of origin to the disposal site. The second group consists of members of the public who may be indirectly exposed to the radionuclides from the material after its disposal.

### Considerations

In applying the dose per unit activity concentration factors provided in this Section, it is important to take account of the following considerations:

- The results of this assessment should only be used for application of the de minimis concept under the London Convention, since the assumptions used to determine dose per unit activity concentration factors are only appropriate in this context;

- It is only necessary to consider those radionuclides which the national regulatory authority considers relevant;

- The calculation presented in this section should be performed using the activity concentrations of radionuclides present in the candidate material, irrespective of their origin.

### Exposure pathways

The following exposure pathways have been considered for members of the crew:

- External irradiation;

- Inhalation of dusts which may be resuspended from the dried surface of the material; and

- Ingestion of the material to be disposed of.

- The following exposure pathways have been considered for members of the public:

- External exposure to radionuclides deposited on beach sediments;

- Ingestion of seafood containing radionuclides derived from the material dumped;

- Inhalation of seaspray; and

- Inhalation of dust from resuspended beach sediments.

Table 3.3 provides an indication of the relative importance of the exposure pathways for each candidate material. The screening approach uses all the exposure pathways without regard to the likelihood of the exposure pathways for the particular candidate material.

### Procedure for Calculating Individual Dose

Doses per unit activity concentration to calculate annual individual doses for selected radionuclides are given in Table 3.1. These factors are in units of µSv a-1 per Bq kg-1dry weight and were calculated using Equations I.1 to I.4 and I.6 to I.18 of Appendix I and generic parameter values given in Appendix II and assuming a unit activity concentration of 1 Bq kg-1 dry weight in the candidate material. For members of the public, doses per unit activity concentration for both adults and infants, aged 1 to 2 years, were calculated. For each radionuclide, the values reported in Table 3.1 is the more restrictive value calculated for the two age groups.

The first step in the procedure is to calculate, for a given radionuclide j, the annual individual dose to a crew member (Eind, crew(j)) and to a member of the public (Eind, public(j)). This is given by the product of the dose per unit activity concentration (Dind, crew(j) and Dind, public(j) for the crew and members of the public respectively) from Table 3.2. and the activity concentration of that radionuclide (C(j), in Bq kg-1 dry weight) in the candidate material:

- Eind, crewj=Dind, crewj⋅Cj	(1)

- Eind, publicj=Dind, publicj⋅Cj	(2)

- The next step is to sum the doses calculated using equations (1) and (2) over all radionuclides included in the assessment for each group of individuals separately.

- The total annual individual dose for the crew is given by:

- Eind,crew=jNEind, crewj,	(3)

where N is the number of radionuclides considered for the assessment. Equation (3) can also be written as:

Eind, crew=Eind, crew1+Eind,crew2+......+Eind,crewN.

- The total annual individual dose to a member of the public is given by:

- Eind, public=M1´109⋅jNEind,  publicj	(4)

which can also be written as:

- Eind, public=M1´109⋅(Eind, public1+Eind, public2+......+Eind, publicN),

where M is the mass of material disposed of in a year at a single dumping site in units of kg a-1 dry weight. In calculating the dose per unit activity concentration factors for this exposure group, a mass of 1 ´ 109 kg a-1 dry weight disposed of per dumping site has been assumed. The parameter M is included explicitly in equation (4) to allow the individual dose to members of the public to be scaled by the actual mass of material dumped annually.

These calculations provide two estimates of the total annual individual dose - for members of the crew and the public respectively for comparison with the exemption criterion for individual doses. The two groups are assumed to consist of different people. Thus, it would not be reasonable to sum these doses together. The doses should be compared separately with the radiological criterion of 10 µSv a-1 specified in IAEA-TECDOC-1068.

### Procedure for Calculating Collective Dose

Dose per unit activity concentration to calculate annual collective doses for selected radionuclides are given in Table 3.2. These factors are in units of manSv a-1 per Bq kg-1 dry weight and were calculated using Equations I.5 and I.19 to I.21 of Appendix I and generic parameter values given in Appendix II and assuming a unit activity concentration of 1 Bq kg-1 dry weight in the candidate material. Doses per unit activity concentration for members of the public given in Table 3.3 were calculated for adults only, as no distinction is made between different age groups when collective doses are calculated.

- The first step in the procedure is to calculate, for a given radionuclide j, the annual collective dose to the crew (Ecoll, crew(j)) and to the members of the public (Ecoll, public(j)). This is given by the product of the dose per unit activity concentration (Dcoll, crew(j) and Dcoll, public(j) for the crew and members of the public respectively) from Table 3.3. and the activity concentration of that radionuclide (C(j), in Bq kg-1 dry weight) in the candidate material:

- Ecoll, crewj=Dcoll, crewj⋅Cj	(5)

- Ecoll, publicj=Dcoll, publicj⋅ Cj	(6)

The next step is to sum the collective doses calculated using equations (5) and (6) over all radionuclides.

- The total collective dose to the crew is:

- Ecoll, crew=jNEcoll, crewj	(7)

which can also be written as:

- Ecoll, crew= Ecoll, crew1+ Ecoll, crew2+......+ Ecoll, crewN.

- The total collective dose for members of the public is calculated in a similar way to the total individual dose (equation (4)) using the equation:

- Ecoll, public= M1´109⋅jNEcoll,  publicj	(8)

which can also be written as:

- Ecoll, public= M1´109⋅(Ecoll, public1+ Ecoll, public2+......+ Ecoll, publicN),

where M is the mass of material disposed of in a year at a single dumping site in units of kg a-1 dry weight.

The final step is to sum the total collective dose to the crew (Ecoll, crew) and members of the public (Ecoll, public):

- Ecoll= Ecoll, crew+ Ecoll, public	(9)

The total annual collective dose thus calculated should then be compared with the applicable radiological criterion of 1 manSv a-1 specified in IAEA-TECDOC-1068.

Summary of Equations for Screening Assessment 1. Individual dose to crew for one radionuclide Eind, crewj=Dind, crewj⋅Cj (1) 2. Individual dose to public for one radionuclide Eind, publicj=Dind, publicj⋅Cj (2) 3. Individual dose to crew Eind,crew=jNEind, crewj10mSva−1 (3) 4. Individual dose to public Eind, public=M1´109⋅jNEind, publicj10mSva−1 (4) 5. Collective dose to crew for one radionuclide Ecoll, crewj=Dcoll, crewj⋅Cj (5) 6. Collective dose to public for one radionuclide Ecoll, publicj=Dcoll, publicj⋅ Cj (6) 7. Collective dose to crew Ecoll, crew=jNEcoll, crewj (7) 8. Collective dose to public Ecoll, public= M1´109⋅jNEcoll, publicj (8) 9. Total collective dose Ecoll, tot=Ecoll, crew+Ecoll, public≤1 manSv a-1 (9)

### Comparison with Decision Criteria

If both individual doses to members of the crew and the public are less than or equal to a value of 10 µSv a-1, the candidate material is suitable for further consideration for disposal under the London Convention, on the basis of this screening assessment. There is, however, an additional condition for de minimis which relates to the collective dose. The candidate material must also comply with the condition that the collective dose is less than or equal to 1 manSv a-1 from disposal activities in one year before it can be designated as de minimis.

If anyone of these criteria is exceeded the candidate material cannot be considered de minimis on the basis of this screening approach. There are two options at this point. The first is to make a decision not to take the assessment further and not to dispose of this material at sea. The second option is to consult modelling and radiological protection specialists to conduct a more detailed specific assessment which takes account of local conditions.

## Detailed Specific Assessment

Candidate materials not determined to be de minimis by the screening approach can be further evaluated in a detailed specific assessment. This would require the services of modelling and radiation protection specialists. An example procedure which could be used for undertaking a detailed specific assessment is provided in Appendices I and II. This procedure should take further account of local conditions.

TABLE 3.1. INDIVIDUAL DOSE PER UNIT ACTIVITY CONCENTRATION IN CANDIDATE MATERIAL (µSv a-1 per Bq kg-1 dry weight)

| Radionuclide | Crew (Dind, crew(j)) | Members of the public (Dind, public(j)) |
| --- | --- | --- |
| Artificial |  |  |
| Ag-110m | 5.4 10-2 | 9.8 10-3 |
| Am-241 | 2.0 10-3 | 1.2 10-3 |
| Ca-45 | 7.1 10-6 | 4.4 10-6 |
| Ce-141 | 7.0 10-5 | 1.6 10-4 |
| Ce-144 | 6.7 10-5 | 9.7 10-4 |
| Cl-36 | 9.3 10-6 | 2.9 10-6 |
| Cm-242 | 1.2 10-4 | 9.5 10-5 |
| Cm-243 | 1.9 10-3 | 1.5 10-3 |
| Cm-244 | 1.2 10-3 | 9.5 10-4 |
| Co-57 | 3.5 10-5 | 3.2 10-4 |
| Co-58 | 1.8 10-2 | 2.3 10-3 |
| Co-60 | 6.2 10-2 | 6.8 10-3 |
| Cr-51 | 2.1 10-4 | 6.0 10-5 |
| Cs-134 | 2.8 10-2 | 3.8 10-3 |
| Cs-137 | 8.1 10-3 | 3.1 10-3 |
| Fe-55 | 3.3 10-6 | 1.3 10-4 |
| Fe-59 | 3.0 10-2 | 3.0 10-3 |
| Hg-203 Error: Reference source not found | 1.6 10-3 | 1.3 10-2 |
| I-125 Error: Reference source not found | 1.5 10-4 | 2.9 10-4 |
| I-129 | 1.1 10-3 | 1.8 10-3 |
| I-131 Error: Reference source not found | 4.8 10-3 | 4.4 10-4 |
| Ir-192 | 8.3 10-3 | 2.0 10-3 |
| Mn-54 | 1.5 10-2 | 2.3 10-3 |
| Na-22 | 5.2 10-2 | 5.8 10-6 |
| Nb-95 | 1.5 10-2 | 1.6 10-3 |
| Np-237 | 2.3 10-3 | 6.7 10-3 |
| Pm-147 | 2.6 10-6 | 5.3 10-7 |
| Pu-238 | 2.3 10-3 | 3.8 10-3 |
| Pu-239 | 2.5 10-3 | 4.2 10-3 |
| Pu-241 | 1.1 10-4 | 1.2 10-4 |
| Pu-242 | 2.4 10-3 | 4.0 10-3 |
| Ru-103 | 6.6 10-3 | 5.3 10-4 |
| Ru-106 | 3.2 10-3 | 5.3 10-3 |
| S-35 Error: Reference source not found | 7.7 10-6 | 6.1 10-6 |
| Sb-124 Error: Reference source not found | 3.9 10-2 | 3.0 10-3 |
| Sb-125 Error: Reference source not found | 5.4 10-3 | 1.3 10-3 |
| Se-75 Error: Reference source not found | 2.4 10-3 | 2.5 10-2 |
| Sn-113 Error: Reference source not found | 2.8 10-3 | 8.1 10-2 |
| Sr-85 | 6.6 10-3 | 5.2 10-4 |
| Sr-89 | 2.9 10-5 | 7.3 10-5 |
| Sr-90 | 3.1 10-4 | 5.1 10-4 |

TABLE 3.1. (CONTINUED)

| Radionuclide | Crew (Dind, crew(j)) | Members of the public (Dind, public(j)) |
| --- | --- | --- |
| Tc-99 | 6.4 10-6 | 2.5 10-4 |
| Tl-204 | 1.2 10-5 | 1.7 10-3 |
| Zn-65 | 1.1 10-2 | 6.0 10-3 |
| Zr-95 | 3.0 10-2 | 5.6 10-3 |
| Natural |  |  |
| Pb-210 Error: Reference source not found | 1.9 10-2 | 4.8 10-3 |
| Po-210 | 1.2 10-2 | 1.7 10-3 |
| Ra-224 Error: Reference source not found | 4.5 10-2 | 1.1 10-2 |
| Ra-226 Error: Reference source not found | 6.0 10-2 | 7.3 10-2 |
| Th-228 | 4.6 10-2 | 5.0 10-2 |
| Th-230 | 6.2 10-2 | 7.8 10-2 |
| Th-232 | 7.2 10-2 | 1.1 10-1 |
| U-234 | 4.9 10-4 | 2.4 10-3 |
| U-235 | 2.3 10-2 | 7.4 10-2 |
| U-238 | 3.4 10-3 | 1.3 10-2 |
| U-238s | 6.4 10-2 | 8.5 10-2 |

TABLE 3.2. COLLECTIVE DOSE PER UNIT ACTIVITY CONCENTRATION IN CANDIDATE MATERIAL (man Sv a-1 per Bq kg-1 dry weight)

| Radionuclide | Crew (Dcoll, crew(j)) | Members of the public (Dcoll, public(j)) |
| --- | --- | --- |
| Artificial |  |  |
| Ag-110m | 5.4 10-6 | 6.4 10-4 |
| Am-241 | 2.0 10-7 | 9.4 10-5 |
| Ca-45 | 7.1 10-10 | 2.1 10-7 |
| Ce-141 | 7.0 10-9 | 5.5 10-7 |
| Ce-144 | 6.7 10-9 | 3.7 10-6 |
| Cl-36 | 9.3 10-10 | 1.6 10-8 |
| Cm-242 | 1.2 10-8 | 7.8 10-6 |
| Cm-243 | 1.9 10-7 | 1.1 10-4 |
| Cm-244 | 1.2 10-7 | 8.4 10-5 |
| Co-57 | 3.5 10-9 | 1.4 10-6 |
| Co-58 | 1.8 10-6 | 8.5 10-6 |
| Co-60 | 6.2 10-6 | 2.8 10-5 |
| Cr-51 | 2.1 10-8 | 2.2 10-7 |
| Cs-134 | 2.8 10-6 | 8.7 10-5 |
| Cs-137 | 8.1 10-7 | 6.2 10-5 |
| Fe-55 | 3.3 10-10 | 1.2 10-5 |
| Fe-59 | 3.0 10-6 | 5.9 10-5 |
| Hg-203 | 1.6 10-7 | 5.6 10-4 |
| I-125 | 1.5 10-8 | 2.0 10-5 |
| I-129 | 1.1 10-7 | 1.7 10-4 |
| I-131 | 4.8 10-7 | 1.3 10-5 |
| Ir-192 | 8.3 10-7 | 6.3 10-6 |
| Mn-54 | 1.5 10-6 | 8.0 10-6 |
| Na-22 | 5.2 10-6 | 8.8 10-8 |
| Nb-95 | 1.5 10-6 | 5.0 10-6 |
| Np-237 | 2.3 10-7 | 3.6 10-4 |
| Pm-147 | 2.6 10-10 | 4.1 10-8 |
| Pu-238 | 2.3 10-7 | 3.3 10-4 |
| Pu-239 | 2.5 10-7 | 3.6 10-4 |
| Pu-241 | 1.1 10-8 | 9.8 10-6 |
| Pu-242 | 2.4 10-7 | 3.5 10-4 |
| Ru-103 | 6.6 10-7 | 3.2 10-5 |
| Ru-106 | 3.2 10-7 | 3.9 10-4 |
| S-35 | 7.7 10-10 | 2.6 10-7 |
| Sb-124 | 3.9 10-6 | 8.1 10-5 |
| Sb-125 | 5.4 10-7 | 4.5 10-5 |
| Se-75 | 2.4 10-7 | 1.3 10-3 |
| Sn-113 | 2.8 10-7 | 3.0 10-3 |
| Sr-85 | 6.6 10-7 | 1.7 10-6 |
| Sr-89 | 2.9 10-9 | 6.0 10-7 |
| Sr-90 | 3.1 10-8 | 6.6 10-6 |
| Tc-99 | 6.4 10-10 | 2.3 10-5 |
| Tl-204 | 1.2 10-9 | 5.9 10-5 |
| Zn-65 | 1.1 10-6 | 4.4 10-4 |
| Zr-95 | 3.0 10-6 | 1.8 10-5 |

TABLE 3.2. (CONTINUED)

| Radionuclide | Crew (Dcoll, crew(j)) | Members of the public (Dcoll, public(j)) |
| --- | --- | --- |
| Natural |  |  |
| Pb-210 | 1.9 10-6 | 4.3 10-4 |
| Po-210 | 1.2 10-6 | 1.6 10-4 |
| Ra-224 | 4.5 10-6 | 3.0 10-4 |
| Ra-226 | 6.0 10-6 | 6.2 10-3 |
| Th-228 | 4.6 10-6 | 1.4 10-3 |
| Th-230 | 6.2 10-6 | 6.2 10-3 |
| Th-232 | 7.2 10-6 | 7.4 10-3 |
| U-234 | 4.9 10-8 | 3.9 10-5 |
| U-235 | 2.3 10-6 | 2.2 10-3 |
| U-238 | 3.4 10-7 | 1.2 10-4 |
| U-238s | 6.4 10-6 | 6.3 10-3 |

TABLE 3.3. POTENTIAL EXPOSURE PATHWAYS DURING AND AFTER THE DUMPING PROCESS

| Candidate materials | External radiation |  | Inhalation |  | Ingestion |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Crew | Public | Crew | Public | Crew | Public |
| Dredged materials | ü ü ü | ü | ü | ü | ü | ü ü ü |
| Sewage sludge | ü ü ü | ü ü | ü ü | ü ü | ü | ü ü ü |
| Fish waste | ü ü ü | ü ü | ü ü | ü ü | ü | ü ü ü |
| Vessels | ü ü ü | ü ü | ü | ü | ü | ü |
| Platforms | ü ü ü | ü ü | ü | ü | ü | ü |
| Inert geological materials | ü ü ü | ü | ü ü ü | ü | ü ü | ü ü ü |
| Organic materials of natural origin | ü ü ü | ü ü | ü ü | ü ü | ü ü | ü ü ü |
| Bulky wastes (steel, concrete, etc.) | ü ü ü | ü | ü ü | ü | ü | ü |

References

| [1] | INTERNATIONAL MARITIME ORGANIZATION, Report of the 19th Consultative Meeting of the Contracting Parties to the Convention on the Prevention of Marine Pollution by Dumping of Wastes and Other Matter, LC 19/10, IMO, London (1997). |
| --- | --- |
| [2] | INTERNATIONAL ATOMIC ENERGY AGENCY, Application of Radiological Exclusion and Exemption Principles to Sea Disposal, IAEA-TECDOC-1068, Vienna (1999). |
| [3] | INTERNATIONAL MARITIME ORGANIZATION, Report of the 21st Consultative Meeting of the Contracting Parties to the Convention on the Prevention of Marine Pollution by Dumping of Wastes and Other Matter, LC 21/13 , IMO, London (1999). |
| [4] | INTERNATIONAL COMMISSION ON RADIOLOGICAL PROTECTION, 1990 Recommendations of the International Commission on Radiological Protection, ICRP Publication 60, Pergamon Press, Oxford and New York (1991). |
| [5] | INTERNATIONAL ATOMIC ENERGY AGENCY, Protection of the environment from the effects of ionizing radiation, A report for discussion, IAEA-TECDOC-1091, Vienna (1999). |
| [6] | INTERNATIONAL MARITIME ORGANIZATION, Resolution LDC 44(14), Report of the Fourteenth Consultative Meeting of the Contracting Parties to the Convention on the Prevention of Marine Pollution by Dumping of Wastes and Other Matter, LC 14/7, IMO, London (1991). |
| [7] | INTERNATIONAL ATOMIC ENERGY AGENCY, International Basic Safety Standards for Protection against Ionizing Radiation and for the Safety of Radiation Sources (jointly sponsored by FAO, IAEA, ILO, OECD/NEA, PAHO and WHO), Safety Series No. 115, IAEA, Vienna (1996). |

APPENDIX I
PROCEDURE for assessing iNDIVIDUAL and collective doses arising from disposals in coastal seas

This Appendix provides a generic procedure for assessing individual and collective doses to the crew and members of the public arising from disposal at sea of material containing radionuclides. The parameters necessary to apply the methodology described in this appendix are identified in Appendix II. Generic parameter values that may be used in the absence of more site-specific information are also provided in that appendix. Doses per unit activity concentration presented in Section 3 of this report have been derived using the procedure described in Appendices I and II.

I.1. Individuals inCLUDED in the Procedure

The methodology described in this appendix has been developed to assess radiation doses to two groups of individuals:

- crew members exposed during transport of material to its dumping site, and

- members of the public most exposed to radionuclides released from the dumped material from the ingestion of marine foods, caught in the vicinity of the dump site, and occupancy on beaches.

While individual doses to the crew can be evaluated assuming that members of this group are adults, the calculation individual doses to members of the public should take account of the difference in age of these individuals. Parameter values in Appendix II, used in the calculation of individual doses to members of the public, are provided for two age groups: adults and infants aged 1 to 2 years.

I.2. Calculation of doses to people exposed during transport of material to its dumping site

I.2.1. Exposure pathways

During shipment of the material and during loading and dumping, the crew of the ship transporting the material to the dumping site can be exposed to the radioactivity in the material through different pathways:

- External irradiation by gamma emitting radionuclides in the material;

- Inhalation of dusts which may be re-suspended from the dried surface of the material;

- Ingestion of the material.

Other exposure pathways may have to be taken into account depending on the actual situation, e.g. skin contamination if the crew on board the ship are likely to handle the material.

I.2.2. Calculation of individual doses to the crew

I.2.2.1. External exposure

The calculation of the dose to the crew due to external irradiation should take account of the properties of the material (density, water contents, etc.) and of the geometry of the ship (load capacity, average distance of crew from the material, thickness of metal for shielding, etc.).

The annual effective dose, from external irradiation, Eext, crew (in Sv a-1), can be calculated using equation (I.1):

Eext, crew=tcrew⋅jCj⋅DFshipj	(I.1)

where

tcrew	is the time the crew is exposed to radioactivity in the material on board of the ship (in h a-1);

DFship(j)	is the dose conversion factor for external irradiation on the ship for radionuclide j in the material (in Sv h-1 per Bq kg-1 dry weight);

C(j)	is the concentration of radionuclide j in the material to be disposed of (in Bq kg-1 dry weight).

The dose conversion factor gives the hourly dose per unit activity concentration in the material. This factor depends on the radionuclide, the geometry of the irradiating source which could be set equal to the geometry of the loading compartment of the ship, and the shielding which, in this case, may be provided by the steel of the ship.

I.2.2.2. Inhalation of dust

The calculation of the annual effective dose from inhalation of dust on board of the ship, Einh, crew (in Sv a-1), can be carried out using equation (I.2):

Einh, crew=tcrewtyearRinh, crew⋅DLship⋅jCj⋅DCinhj	(I.2)

where

tcrew	is the time the crew is exposed to radioactivity in the material on board of the ship (in h a-1); its value is taken to be the same as the value used in the calculation of doses due external exposure;

tyear	is the number of hours in a year; a value of 8760 h a-1 can be used for this parameter;

Rinh, crew	is the inhalation rate of members of the crew (in m3 a-1);

DLship	is the concentration of dust in the air on board ship (in kg m-3).

DCinh(j)	is the dose coefficient for inhalation (in Sv Bq-1);

I.2.2.3. Ingestion of candidate material

Ingestion of the material is possible when the crew comes in close contact with the material or with residues of the material, like, for example, when cleaning the ship. The annual effective dose from ingestion of material, Eing, crew (in Sv a-1), can be calculated using equation (I.3).

Eing, crew=Hdust⋅jCj⋅DCingj	(I.3)

where

Hdust	is the ingestion rate of dust (in kg a-1);

DCing(j)	is the dose coefficient for ingestion (in Sv Bq-1);

I.2.2.4. Total individual dose to crew members

The total annual individual dose to a crew member, Eind, crew (in Sv a-1), is the sum of the doses arising from contributions calculated as indicated in equations (I.1) to (I.3) described above. Thus:

Eind, crew=Eext, crew+Einh, crew+Eing, crew	(I.4)

I.2.3. Calculation of collective doses to the crew

The collective dose for the crews carrying out the dumping operations, Ecoll, crew (in manSv a-1), can be obtained in a straightforward way by multiplying the individual doses which have been calculated as described in Section I.3.2 and the number of crew members which will receive these doses. This is expressed in equation (I.5):

Ecoll, crew=Eext, crew+Einh, crew+Eing, crew⋅Ncrew⋅Nship⋅Nsites	(I.5)

where

Ncrew	is the number of crew members involved in the dumping operations for a single dumping site;

Nsites	is the number of dumping sites in operation for a single nation;

Nship	is the number of ships performing dumping operations for a single dumping site.

I.3. Calculation of doses to members of the public exposed as a result of releases of radionuclides from dumped material

I.3.1. Exposure pathways

Radionuclides released from the dumped material disperse in the water and become adsorbed on sediments some of which deposit on the shore. Radionuclides will then be present in both sediments and the water itself. They may be taken up by marine animals and plants and thus enter the human food chain.

Members of the public who live in the coastal area in the vicinity of a dumping site may be exposed to radionuclides in the dumped material through four main pathways:

- External exposure from the radionuclides which have been deposited on the beach;

- Inhalation of sea spray which contains radionuclides;

- Inhalation of dust from resuspended sediment of the beach which contains radionuclides.

- Ingestion of seafood caught in the area around the dumping site.

FIG. I.I. Diagram of model used to simulate dispersion of radionuclides.

I.3.2. Calculation of radionuclide concentrations in water, sediment and marine biota

Many factors can affect the dispersion of radionuclides in the area near the dumping site. These include the physical and chemical form of the radionuclides released, the mode and location of the disposal, the characteristics of the receiving environment, sedimentary processes and marine biological productivity. Assessments of the impact of radioactive releases that are subject to explicit authorisation and control generally require the use of models with site-specific data and assumptions. For the purpose of the procedure described in this appendix it is adequate to use a generic approach based on assumptions that lead to conservative estimates of the radiation doses received by members of the public.

In this procedure disposal of waste into the sea is assumed to occur in shallow (less than 100 m deep) well mixed coastal waters covering. The dumping activity is assumed to take place a few kilometers off the coast so the actual shape of the coastline does not influence the dispersion significantly. Dumping in coastal areas with complex features might require an assessment based on site specific information on coastal properties and seawater dispersion.

Candidate materials may have a range of different physical and chemical forms in which the radionuclides may be bound more or less strongly. For organic materials (such as sewage sludges) the release of radioactivity to the seawater may occur quickly, while radionuclides in geological materials are strongly associated with the solid matter and only released to the seawater at a slow rate. For the purposes of the calculations of this methodology it is assumed that the release of radioactivity from the waste occurs at a constant rate and is completed within one year from the time of dumping. This is a conservative assumption for the case where a substantial fraction of the activity is strongly associated with the solid matter in the waste and the radionuclides are only released to the seawater at a slow rate.

A single-box model is used in this methodology to simulate the dispersion and dilution of the radionuclides in the water column of the region where the material is disposed of under steady-state conditions. The model assumes instantaneous mixing throughout the volume of water in the box. Due to wind and tidal mixing the hydrological conditions in the box vary to the extent that total mixing of the water is a reasonable assumption. The radionuclide concentrations in seawater and sediments predicted by the model are average values over the entire volume of the box and the year of the release. The model assumes instantaneous equilibrium between radionuclides in the soluble phase and those adsorbed on particles suspended in the water column and on particles in the top sediment boundary layer. Both kinds of particles are assumed to be in close contact with the water column. The association of radionuclides between water and sediment is described by element dependent sediment distribution coefficients, Kd. Removal of radioactivity from the water column to the sediment due to scavenging processes is not considered. A diagram of the dispersion model used in this procedure is given in Figure I.I. Average conditions for the environmental system are reasonable since it might not be possible to define the exact locations within the water region of, for example, dumping sites, fishing grounds, coastal areas for beach occupancy. When calculating radionuclide concentrations in fish the use of average values is justified because it takes account of fish migration in the region.

The activity concentration of a radionuclide in the box is dependent on the rate of input of the radionuclide into the box, its radioactive decay and its dispersion. The rate constant due to dispersion, ldis, (in a-1) is the reciprocal of the mean residence time in the water region and is obtained from the equation:

λdis=FV	(I.6)

where

V	is the volume of seawater in the box (in m3);

F	is the average flux of water through the coastal region (in m3 a-1).

The average rate of input of activity of radionuclide j, Q(j) (in Bq a-1), is obtained from the equation:

Qj=M⋅Cj	(I.7)

where

M	is the mass of candidate material dumped (in kg a-1);

C(j)	is the average concentration of radionuclide j in the candidate material (in Bq kg-1 dry weight).

The equilibrium concentration of radionuclide j in the box, CBOX(j) (in Bq m-3), is given by:

CBOXj=QjVλj+λdis	(I.8)

where

l(j)	is the radioactive decay constant for radionuclide j (in a-1).

The concentration CBOX(j) in the box includes radioactivity in dissolved phase of seawater and radioactivity associated with the suspended sediment particles and with the sediment boundary layer. The concentration by volume of radionuclide j in the dissolved phase of seawater, CDW(j) (in Bq m-3), is given by:

CDWj=CBOXj1+Kdj⋅S+LB⋅ρBD	(I.9)

where

Kd(j)	is the sediment distribution coefficient for radionuclide j (in m3 kg-1);

S	is the suspended sediment concentration (in kg m-3);

LB	is the thickness of the sediment boundary layer (in m);

rB	is the density of the sediment boundary layer (in kg m-3);

D	is the average depth of the water column (in m).

The concentration, by weight, in the suspended particles, CP(j) (in Bq kg-1), is obtained from:

CPj= Kdj⋅ CDWj	(I.10)

The total concentration in seawater, CW(j) (in Bq m-3), is given by:

CWj= 1+Kdj⋅SCDWj	(I.11)

Transfer of radioactivity to biota is calculated from the concentration of dissolved radioactivity in the water using element-dependent concentration factors for biological material. Radionuclide concentrations in fish and shellfish are obtained from the concentrations in dissolved phase seawater, CDW(j) multiplied by the appropriate concentration factors:

CBj,k=CFBj,k⋅CDWj	(I.12)

where

CB(j,k)	is the activity concentration of radionuclide j in marine biota k (in Bq kg-1 fresh weight);

CFB(j,k)	is the concentration factor of radionuclide j in marine biota k (in Bq kg-1 per Bq m-3).

For the calculation of the external exposure from contaminated beach sediments the radionuclide concentration in coastal sediment is assumed to be a factor of 10 lower than that in suspended particles in the water column [I.1]. The surface contamination in the coastal sediment of radionuclide j, CS(j) (in Bq m-2), is obtained from the equation:

CSj=CPj⋅ρS⋅ds10	(I.13)

where

rS	is the density of coastal sediment (in kg m-3);

dS	is the effective thickness of coastal sediment (in m).

Airborne fine coastal sediment particles considered for the inhalation pathway are assumed to have characteristics similar to the suspended particles in the water column.

I.3.3. Calculation of individual doses to members of the public

I.3.3.1. External exposure

The annual effective dose to members of the public from external exposure to radioactivity on the shore, Eext, public (in Sv a-1) can be calculated using equation (I.14):

Eext, public=tpublictyear⋅jCSj⋅DFgrj	(I.14)

where

tpublic	is the time spent by members of the public on the beach in a year (in h a-1);

tyear	is the number of hours in a year (8760 h a-1);

DFgr(j)	is the dose conversion factor for ground contamination of radionuclide j (in Sv a-1 (Bq m-2)-1);

CS(j)	is the surface contamination of radionuclide j in the beach sediments (in Bq m-2);

I.3.3.2. Doses from inhalation of resuspended beach sediments

The dose from inhalation of resuspended sediments from the beach, Einh, shore, public can be calculated using equation (I.15):

Einh, shore, public=tpublictyearRinh, public⋅DLshore⋅jCPj⋅DCinhj	(I.15)

where

Rinh, public	is the inhalation rate of members of the public (in m3 a-1);

DLshore	is the concentration of dust in the air on the shore due to resuspension of beach sediments (in kg m-3).

DCinh(j)	is the dose coefficient for inhalation for radionuclide j (in Sv Bq-1);

The radionuclide concentration in sediment CP(j) is calculated using equation (I.10). The occupancy of people on the beach, tpublic (in h a-1) is taken to be the same as the value used in the calculation of doses due external exposure.

I.3.3.3. Doses from inhalation of sea spray

The annual dose to members of the public from inhalation of airborne sea spray on the shore can be calculated using equation (I.16):

Einh, spray, public=tpublictyearRinh, public⋅Csprayρw⋅jCWj⋅DCinhj	(I.16)

where

Cspray	is the concentration of sea spray in the air (in kg m-3)

rW	is the density of seawater (in kg m-3); a value of 1000 kg m-3 can be used for this parameter;

CW(j)	is the concentration of radionuclide j in sea water (in Bq m-3);

The radionuclide concentration in seawater CW(j) is calculated using equation (I.11). The occupancy of people on the beach, tpublic (in h a-1) is taken to be the same as the value used in the calculation of doses due external exposure. The inhalation rate, Rinh, public (in m3 a-1) and the dose coefficient for inhalation, DCinh(j) (in Sv Bq-1) are the same as those used to calculate doses due to inhalation of resuspended sediments.

I.3.3.4. Doses from ingestion of fish and shellfish

The total annual effective dose from the ingestion of fish and shellfish, Eing,public (in Sv a-1) can be calculated using equation (I.17):

Eing,public=kHBk⋅jCBj,k⋅DCing,jj	(I.17)

where

HB(k)	is the consumption rate for marine biota k (in kg a-1);

DCing(j)	is the dose coefficient for ingestion of radionuclide j (in Sv Bq-1).

CB(j,k)	is the concentration of radionuclide j in marine food k (in Bq kg-1 fresh weight) as calculated using equation (I.12).

I.3.3.5. Total individual dose to members of the public

The total individual dose to a representative member of the public, Eind,public (in Sv a-1) exposed to radionuclides released from materials dumped at sea, may be estimated as the sum of the dose contributions calculated as indicated in equations (I.14) – (I.17). Thus:

Eind, public=Eext, public+Einh, shore, public+Einh, spray, public+Eing, public	(I.18)

I.3.4. Calculation of collective doses

The collective dose for members of the public is the combination of collective doses from exposures on the beach and from consumption of seafood.

I.3.4.1. Collective dose from beach occupancy

The collective dose which results from time spent on the beach, is calculated from the individual doses to members of the public from external and inhalation exposure pathways as specified in equations I.14 to I.16, modified to account for the time average members of the public spend on the beach. The collective dose for people spending time on the shore (in manSv a-1) is given by:

Ecoll, shore, public=Eext,public+Einh,shore,public+Einh,spray,publicOcoll, public⋅Lshore⋅Nsitestpublic	(I.19)

where

Ocoll, public	is the annual collective occupancy time per unit length of coastline (in man h m-1 a-1);

Lshore	is the length of coastline affected by dumping operation on singe site (in m);

Nsites	is the number of dumping sites in operation.

The product of Ocoll, shore and Lshore gives the collective occupancy by members of the public on the coastline of one dumping site. The the time spent on the shore by a member of the public in one year, tpublic (in h a-1), is the same value used in equations (I.14) to (I.16).

In a more refined assessment, the collective doses for each dumping site could be assessed separately. The collective dose could then be obtained from summing over the collective doses calculated separately for each dumping site.

I.3.4.2. Collective dose from seafood consumption

The collective dose from seafood consumption, Ecoll, ing, public (in manSv a-1) is calculated from the entire seafood catch from the coastal region in question, taking account of fraction that is used for human consumption by man. This is expressed in equation. (I.20):

Ecoll, ing, public=NsiteskfBk⋅NBk⋅jCBj,k⋅DCingj,k	(I.20)

where

NB(k)	is the annual amount of marine biota k caught in the area which is affected by a single dumping site (in kg a-1);

fB(k)	is the fraction of marine biota k used for human consumption;

Nsites	is the number of dumping sites in operation.

The product of fB(k) and NB(k) is assumed to be the collective annual consumption rate of marine biota k for members of the public that are exposed to radioactivity from material disposed at one dumping site. As in the case of the calculation of collective dose from beach occupancy, a more refined assessment could take account of the characteristics of each site.

I.3.4.3. Total collective dose

The total collective dose, Ecoll, public, is obtained by summing the two contributions from exposures on the beach and seafood consumption:

Ecoll, public=Ecoll, shore, public+Ecoll, food, public	(I.21)

I. 4. CONSERVATIVE NATURE OF THE PROCEDURE

A generic procedure, such as the one described in this appendix, must be based on conservative assumptions in order to avoid to underestimate the radiation doses arising from the disposal of candidate material at sea. A description of the conservative approach adopted to calculate individual and collective doses for crew and members of the public is given below.

In the assessment of doses to the crew, conservative assumptions were adopted for the geometry of the load on the ship in the calculation of the dose from external irradiation. In addition it is unlikely, in many circumstances that members of the crew inhale and ingest dust which may be released from the load, as it was assumed in the calculations.

Doses for members of the public have been calculated on the basis of the following conservative assumptions:

- It is assumed that all of radioactivity in the dumped material is released into the water. Under normal circumstances, a substantial part would be retained in the material.

- The release is assumed to occur within a single year. Release would normally occur over significantly longer time intervals for most radionuclides.

- After the release, complete and instantaneous mixing is assumed within the ocean area which is under consideration.

- The dumping is considered to take place continuously over the year resulting in steady state (equilibrium) conditions being established.

As a result of these four conservative assumptions the radionuclide concentrations in water are significantly higher than would be in reality. Other conservative assumptions adopted in the procedure include:

- The dumping operations are considered to take place near fishing grounds which are generally avoided. Therefore, inclusion of seafood catch from an area directly near the place of dumping is to be considered conservative. Furthermore, no fish migration outside the area under consideration is assumed.

- The dumping operation is assumed to take place near the coast and in shallow waters while in most cases the dumping operations occur in deeper waters and further off the coast. This results in a conservative estimate of the doses due to exposure to radionuclides deposited on the shore.

In addition, the calculation of dose per unit activity concentrations for collective doses, (see Table 3.3) has been based on the conservative assumption that 10 dumping sites are in operation simultaneously. In general, the number of dumping sites in operation would be lower.

I.5. Limitations of THE methodology

In the approach described in this Appendix disposals are assumed to take place in shallow coastal waters. The transfer of radionuclides from dumping in the deep sea is considered to be too complex to be assessed by simple modelling techniques, since detailed oceanographic modelling of water movements are required. It is also considered that deep sea dumping is likely to occur infrequently such that a generic model would be of less value to national authorities than one dealing with coastal waters. A number of approaches exist for assessing the radiological impact of deep sea dumping of materials. Although developed for the purpose of evaluating the effects of dumping radioactive materials, the methodologies themselves may be of value in assessing the radiological impact of candidate materials dumped in the context of the London Convention 1972. Individual dose to members of the public from waste disposal on the ocean seabed may be assessed, for instance, by applying the model described in Ref. [I.2]. Collective dose to members of the public may be estimated using the results presented in Ref. [I.3].

References

| [I.1] | SIMMONDS, J.R., LAWSON, G., MAYALL, A., Methodology for Assessing the Radiological Consequences of Routine Releases of Radionuclides to the Environment, EUR 15 760, European Commission, Luxembourg (1995). |
| --- | --- |
| [I.2] | INTERNATIONAL ATOMIC ENERGY AGENCY, Definition and Recommendations for the Convention on the Prevention of Marine Pollution by Dumping of Wastes and other Matter, 1972, 1986 Edition, Safety Series No. 78, IAEA, Vienna (1986). |
| [I.3] | DEPARTMENT OF THE ENVIRONMENT, DOE Report No. DOE/RW/86-136 (1986). Calculations of the radiological impact of disposal of unit activity of selected radionuclides. |

aPPENDIX II
parameters including default values and suggested information sources

This Appendix describes the parameters which are required to perform the calculations described in Appendix I and provides generic values for them. For element or radionuclide dependent parameters, values are provided for a number of radionuclides which are of radiological significance in the context of dumping. Alternative sources from which values may be obtained are listed in Table II.I. All generic values recommended in this Appendix are given in Tables II.III–II.VIII. For the parameters used in the calculation of individual doses to members of the public, values for adults and infants aged 1 to 2 years are provided (see Table II.VII).

II.1. Use of GENERIC data

It is recommended that site specific data be used whenever possible. Generic data, of the type provided in this Appendix, should generally only be used whenever site specific data are not available. Where possible, site-specific information on the characteristics of the receiving coastal environment and specific information about the dumping operations should be used. For example, the size of the vessel transporting the waste to the dumping site, the number of crew per ship and the time for transport of the waste from port to the dumping site should be defined. Information about local human habits concerning beach occupancy and ingestion of seafood, for example, would also be useful if a more detailed site-specific assessment was needed.

II.2. Inclusion of Progeny

When performing an assessment of radiation doses such as that described in Appendices I and II, the contribution to the dose from progeny in the candidate material and in other environmental materials should be taken into account, if there is no reason to believe that the progeny originate from any additional source than the decay of the parent radionuclides. Table II.II lists those progeny which should be included with the respective parent radionuclide. For some radionuclides only some progeny of the decay chain have been included. Notes to the Table provide an explanation of the considerations adopted to select the progeny. Table II.II also gives the weighting factors used to determine the contribution of the progeny to the dose coefficients. If not otherwise indicated in the table, the weighting factor assumed is 1 (corresponding to a 100% decay into the progeny). Weighting factors smaller than 1, resulting from the decay into two progenies, are provided in brackets next to the progeny.

Values for the dose coefficients for external irradiation presented in Table II.V include the contribution of the progeny. This has been achieved by adding the dose coefficients of the progeny, multiplied by weighting factors given in Table II.II, to the dose coefficient of the parent radionuclide. When doses due to external irradiation are calculated using the values given in Table II.V the contribution of the progeny need not be considered separately.

II.3. Parameters and default values

II.3.1. Parameters related to the dumping operation

II.3.1.1. Annual mass of material dumped, M

The mass of the candidate material to be disposed of annually is a key input factor in determining the radionuclide concentrations in environmental media. If there are no site specific data, a value of 1 × 109 kg a-1 is suggested as a reference quantity for a single dumping site which involves many operations and large volumes of material, such as dredging material from a major harbour (see Table II.VI).

II.3.2. Parameters related to radionuclides or elements

II.3.2.1. Decay constants ë(j)

Decay constants of a number of radionuclides which are of radiological significance in the context of dumping are given in Table II.III. The values were calculated using half-lives of the radionuclides taken from ICRP Publication 38 [II.1]

II.3.2.2. Sediment distribution coefficients, Kd(j)

Kds are used to calculate the partitioning of radionuclide concentration between sea water and sediments. Kd values for each radionuclide are provided in Table II.IV. These values were taken from IAEA Technical Report Series No. 247 [II.2] and are recommended, unless different Kd values, applicable to the particular marine environment for which the assessment is made, are available.

II.3.2.3. Concentration factors for marine biota, CFB(j,k)

Element specific concentration factors are used to calculate the concentration of radionuclides in marine biota from the concentration in the sea water. For the purpose of the procedure described in Appendix I the types of marine biota included in the assessment are fish and shellfish (crustaceans and molluscs). Recommended values are taken from IAEA TRS 247 [II.2] and are provided in Table II.IV. In IAEA TRS 247 concentration factors for crustaceans and molluscs are provided separately. For each element the values listed in Table II.IV are the higher of the two values given in IAEA TRS 247.

II.3.3. Dose coefficients for external and internal irradiation

II.3.3.1. Dose conversion factors for external irradiation from the ship load, DFship(j)

Default values for the dose conversion factors for external irradiation on board of the ship are provided in Table II.V. The values have been calculated with a standard software package [II.3], include the contribution of progeny as given in Table II.II and are based on the following assumptions:

- the ship load is 1000 m3 (length 56 m, width 6 m, height 3 m);

- the average position of the crew member is on a deck 2 m above the load;

- shielding is provided by steel with a density of 7.6 g cm-3 and a thickness of 2 cm (one or two decks) between the upper side of the load and the crew member;

- doses are calculated for a point 1 m above the deck on which the crew member is located (i.e. for a point 3 m above the upper surface of the load);

- the density of the material to be dumped is taken to be 1.5 g cm-3.

A diagram of the geometry used in the calculation of the dose conversion factors is given in Figure II.1.

It is recommended that the dose conversion factors provided in Table II.IV are used if no specific data of the ships are known. If, however, values for the parameters listed above can be identified more precisely, dose conversion factors for external irradiation on board of the ship can be calculated using standard software programs.

II.3.3.2. Dose conversion factors for external irradiation on the beach, DFgr(j)

Default values for dose conversion factors for external irradiation on the beach are provided in Table II.IV. They relate to a surface contamination and are therefore given in Sv a-1 per 
Bq m-2. These values are taken from US EPA Federal Guidance Report No. 12 [II.4] and are based on the conservative assumption that the radionuclides which are washed ashore will be deposited mainly on the surface of the beach. These coefficients include a skin component and the contribution from the ingrowth of progeny according to Table II.II

II.3.3.3. Dose coefficients for ingestion, DCing(j), and inhalation, DCinh(j)

Dose coefficients for ingestion and inhalation for adults and infants (age 1 to 2 years) are provided in Table II.V. They are taken from the IAEA Basic Safety Standards [II.5]. This reference is a standard source of dose coefficients for intakes of radionuclides and the use of other references is not recommended.

II.3.4. Parameters related to the box model

II.3.4.1. Volume of box in the dispersion model, V

The volume of the box used for modelling the dispersion of radionuclides is an important parameter in the calculation of activity concentrations in the region where material is disposed of. The volumes of boxes used to simulate the dispersion of radioactive material released from nuclear power plants in compartmental models for the assessment of the radiological consequences of routine radioactive discharges into North European waters are in the range 1 × 108 m3 to 5 × 109 m3 [II.6]. Although these compartmental models were not developed to predict the dispersion of candidate material disposed at sea in the context of the London Convention 1972, they provide valuable information on which assumptions on the values to be adopted for the assessment procedure described in Appendix I can be made. For a generic assessment, the volume of the box can be assumed to be equal to 2 × 109 m3 (see Table II.VI).

II.3.4.2. Depth of water column, D,

These two parameters are used to calculate radionuclide concentrations in the dissolved phase of seawater. Depths of dispersion boxes in compartmental models for the assessment of the radiological consequences of routine radioactive discharges into North European waters are in the range 10 m to 40 m [II.6]. For the purposes of the assessment described in Appendix I, a generic value of 20 m for the depth of the water column is recommended (see Table II.VI).

FIG. II.I. Exposure geometry for crew member on ship, external irradiation.

II.3.4.2. Depth of water column, D, and thickness of sediment boundary layer, LB

This parameter is also used to calculate radionuclide concentrations in the dissolved phase of seawater. The value adopted in compartmental models for the assessment of the radiological consequences of routine radioactive discharges into North European waters is 0.1 m [II.6]. Conservatively a generic value 0.01 m can be used for the procedure described in Appendix I. Since the thickness of the sediment boundary layer is much smaller than the depth of the water column, it can be assumed that the depth of the box used in the dispersion model is equal to the depth of the water column. This means that the surface area of the default box used in this methodology is 100 km2.

II.3.4.4. Flux through the box, F

The flux of seawater through the box is necessary for the calculation of activity concentrations in seawater, suspended sediments particles and coastal sediments, and the value should be determined site specifically. Fluxes through dispersion boxes, with a depth greater than 10 m, in compartmental models for the assessment of the radiological consequences of routine radioactive discharges into North European waters are between 4 × 1010 m3 a-1 and 1.6 × 1011 m3 a-1  [II.6]. Conservatively, the lower value of this range (4 × 1010 m3 a-1) can be used for generic assessments (see Table II.VI), which for a box of 2 × 109 m3 in volume corresponds to a residence time of about 3 weeks.

II.3.4.5. Suspended sediment load in the water column, S

The suspended sediment load in the water column is necessary to calculate the activity concentration in seawater and suspended sediments. Suspended sediment loads in North European waters range from 1 × 10-4 kg m-3 to 6 × 10-3 kg m-3 [II.6]. If no site specific data are available a generic value of 3 × 10-3 kg m-3 may be used (see Table II.VI).

II.3.4.6. Bulk density in suspended sediments, rS, and in boundary layer sediments, rB

The bulk density in sediments is necessary to calculate the radioactivity concentration on the shore from the concentration in the water. For the purpose of the procedure described in Appendix I a generic value of 1.5 g cm-3 may be used for both densities (see Table II.VI).

II.3.4.7. Effective thickness of coastal sediment, dS

The effective thickness of coastal sediment is necessary to calculate the radioactivity concentration on the shore from the concentration in the water. A generic value of 0.1 m may be used (see Table II.VI). This value is used in compartmental models for the assessment of the radiological consequences of routine radioactive discharges into the North European waters  [II.6].

II.3.5. Parameters related to inhalation pathways

II.3.5.1. Dust concentration  in air on board ship, DLship

The concentration of dust in air on board the ship will depend on the nature of the material, whether the ship has an open or a closed loading compartment and whether the surface of the material is or will become dry during the shipment. This parameter will be difficult to establish on a site specific basis. For the purpose of this methodology a concentration of inhalable dust in the air on board of the ship of 2.5 10-10 kg m-3 [II.7] can be assumed (see Table II.VII).

II.3.5.2. Dust concentration in the air on shore, DLshore

It is recommended that the same generic value of 2.5 10-10 kg m-3 as the dust concentration in air on board ship should be used for the concentration of inhalable resuspended dust on the shore [II.7], unless site specific data are available (see Table II.VII).

II.3.5.3. Concentration of sea spray in the air, Cspray

It is recommended that a generic value of 0.01 kg m-3 be used for the concentration of airborne sea spray (marine aerosols) [II.7] unless site specific data are available (see Table II.VII).

II.3.6. Habit data for calculation of individual doses

II.3.6.1. Time of exposure of the crew on the ship, tcrew

The time per year during which the crew on the ship is exposed to the radioactivity in the material depends on whether the total annual mass of material dumped for a single dumping site, M, can be handled by a single ship or whether several ships are needed in parallel. If the total annual material quantity for one dumping site can be handled by a single ship the exposure time can be calculated from the number of shipments, Nshipment, needed to transport all the material, and the duration of each shipment, tshipment, according to equation (II.2).

tcrew=Nshipment⋅tshipment2	(II.1)

Division by 2 accounts for the fact that exposure occurs only during transportation to the dumping site, but not during the return journey. The duration of each shipment consists of the loading time, the travel time to reach the dumping site, the time required for unloading and the return time. The number of shipments which are necessary in a single year is the ratio of the quantity of material to be disposed of at the dumping site M and the load capacity of the ship Mship:

Nshipment=MMship	(II.2)

The load capacity of the ship should be chosen to fit the proposed dumping practice and should therefore be determined on a site specific basis. In the absence of specific data a default capacity of 1000 m3 can be used. This parameter was used to calculate the conversion factors for external irradiation from the ship load (see Section II.3.3.1).

If the annual mass of material dumped, M, is very large so that more than one ship is necessary, the exposure time can simply be set equal to half of the total working hours per year (1000 h a-1) because the ship sails half of the time loaded. These assumptions and have been adopted in this methodology (see Table II.IV). Dumping 1 × 109 kg material per year, using ships with a capacity of 1000 m3, or 1.5 × 106 kg, would require about 670 shipments per year or more than 3 shipments per day for 200 working days per year.

II.3.6.2. Time of exposure of members of the public on the beach, tpublic

If site specific values for the time per year members of the public spend on the beach are not available, it is recommended that 1600 h a-1 for adults and 1000 h a-1 for infants be used [II.8] (see Table II.VII).

II.3.6.3. Ingestion rate for material on board of the ship, Hdust

The inadvertent intake rate of material during transport on the ship is difficult to establish on a site specific basis. It is recommended that a value of 10 g a-1 be used for this parameter (see Table II.VII). This is based on a value of 20 g a-1 for a full working year and the assumption that the ship sails empty half the time [II.9, II.10].

II.3.6.4. Inhalation rate, Rinh, crew and Rinh, public

It is recommended that a generic inhalation rate of 10500 m3 a-1 for crew members (Rinh, crew) and 8400 m3 a-1 for adult members of the public and 1400 m3 a-1 for infant members of the public (Rinh, crew) should be used [II.11] (see Table II.VII). The value for the crew takes into account the fact that during working a higher breathing rate than annual average applies.

II.3.6.5. Annual consumption of seafood, HB(k)

For adults it is recommended that generic annual consumption rates of 50 kg a-1 of fish and 15 kg a-1 of shellfish be used [II.8]. For infants aged 1 to 2 years, an annual consumption rate of 25 kg a-1 of fish should be used, while no consumption of shellfish should be included [II.8] (see Table II.VII). These values should be used unless site specific data taking account of the dietary habits of the local population are available.

II.3.7. Parameters required for the calculation of collective doses

II.3.7.1. Number of crew members on a ship, Ncrew

The size of a crew on the ship which carries out dumping operations is necessary for the calculation of collective dose to the crew. This parameter should be chosen site specifically. However, if the crew size is not known it is recommended to assume 10 crew members being present on a ship (see Table II.VIII).

II.3.7.2. Number of ships for one dumping site, Nship

If the annual mass of material dumped per dumping site, M, can be handled by a single ship, this parameter should be set equal to 1. Otherwise, the number of ships in operation for a single dumping site needs to be assessed site specifically. It must be chosen in such a way that the product of the number of ships, Nship, the load capacity of a ship, Mship, and the annual number of shipments per ship, Nshipment, equals the annual material quantity, M (see Table II.VIII).

II.3.7.3. Number of dumping sites in operation, Nsites

The number of dumping sites in operation is a parameter which is necessary for the calculation of collective doses for both workers and members of the public. It should be chosen according to a country’s dumping practice. If the number cannot be established, then it is recommended that 10 dumping sites be used (see Table II.VIII).

II.3.7.4. Collective occupancy time on shore area, Ocoll,shore

The occupancy time is necessary for the calculation of collective doses from exposures on the beach. It is recommended that a value of 50 man·h m-1 a-1 be used [II.6] (see Table II.VIII).

II.3.7.5. Length of coastline affected by one dumping site, Lshore

The length of coastline which is affected by one dumping site should be chosen equal to the length of the box model which is used for calculation. Given that the surface area assumed for the box used in the procedure detailed in Appendix I is 100 km2 a generic value of 10 km (10 000 m) can be chosen for this parameter (see Table II.VIII).

II.3.7.6. Production of fish and shellfish, NB(k)

The annual catch of fish and shellfish in the area covered by the box model is necessary for the calculation of the collective doses from seafood consumption. Nominal annual catches in North European waters [II.12] indicate that annual rates for fish are in the range 0.3 to 5 t km-2, while for shellfish they are between 0.03 and 1.7 t km-2. For generic assessments, therefore, annual rates of 10 and 4 t km-2 for fish and shellfish respectively can be used. The surface area of the default box described in this methodology is 100 km2. Annual catches of fish and shellfish are therefore 1000 t and 400 t (see Table II.VIII).

II.3.7.7. Fraction of seafood consumed by the population, fB(k)

Only a fraction of the total seafood caught in the area affected by the dumping is used for human consumption. If no site specific information is available it is recommended that a factor of 0.5 for fish and 0.35 for shellfish be used [II.8] (see Table II.VIII).

TABLE II.I. SUMMARY OF POSSIBLE SOURCES OF INFORMATION FOR THE PARAMETER VALUES REQUIRED IN THE CALCULATIONS

| Parameter | Applicant | Radiation Authority | Dumping Authority | Calculation | Generic Value |
| --- | --- | --- | --- | --- | --- |
| Vessel |  |  |  |  |  |
| Mship, Nship, Ncrew | 4 |  |  |  | 4 |
| tshipment | 4 |  | 4 |  |  |
| Nshipment | 4 |  | 4 | 4 |  |
| tcrew | 4 |  |  | 4 | 4 |
| Nsites | 4 |  | 4 |  | 4 |
| Candidate material |  |  |  |  |  |
| M | 4 |  |  |  | 4 |
| l(j) |  | 4 |  |  | 4 |
| Kd(j), CFB(j,k) |  | 4 |  |  | 4 |
| DFship(j), DFgr(j), DCing(j), DCinh(j) |  | 4Error: Reference source not found |  |  | 4 |
| Box model |  |  |  |  |  |
| V, F | 4 | 4Error: Reference source not found | 4 |  | 4 |
| Q(j), ldis(j) |  |  |  | 4 |  |
| D, LB, dS, S, rS, rB | 4 | 4Error: Reference source not found | 4 |  | 4 |
| CBOX(j), CDW(j), CW(j), CP(j), CS(j), CB(j,k) |  |  |  | 4 |  |
| Exposure to crew |  |  |  |  |  |
| tcrew | 4 |  |  | 4 | 4 |
| Rinh, crew, DLship, Hdust |  | 4Error: Reference source not found |  |  | 4 |
| Exposure to public |  |  |  |  |  |
| tpublic |  | 4Error: Reference source not found | 4 |  | 4 |
| Rinh, public DLshore, Cspray |  | 4Error: Reference source not found |  |  | 4 |
| HB(k), NB(k), fB(k) |  | 4Error: Reference source not found | 4 |  | 4 |
| Ocoll, public Lshore |  | 4Error: Reference source not found | 4 |  | 4 |

TABLE II.II. LIST OF PROGENY WHICH SHOULD BE INCLUDED WITH THEIR PARENT RADIONUCLIDES IN THE ASSESSMENT

| Radionuclide | Progeny |
| --- | --- |
| Ce-144 | Pr-144 |
| Cs-137 | Ba-137m |
| Np-237 | Pa-233 |
| Pb-210 | Bi-210, Po-210 |
| Pu-241 Error: Reference source not found | Am-241 (0.03) |
| Ra-224 | Rn-220, Po-216, Pb-212, Bi-212, Tl-208 (0.36), Po-212 (0.64) |
| Ra-226 | Rn-222, Po-218, Pb-214, Bi-214, Po-214, Pb-210, Bi-210, Po-210 |
| Ru-103 | Rh-103m |
| Ru-106 | Rh-106 |
| Sb-125 | Te-125m (0.23) |
| Sn-113 | In-113m |
| Sr-90 | Y-90 |
| Th-228 | Ra-224, Rn-220, Po-216, Pb-212, Bi-212, Tl-208 (0.36), Po-212 (0.64) |
| Th-230 | Ra-226, Rn-222, Po-218, Pb-214, Bi-214, Po-214, Pb-210, Bi-210, Po-210 |
| Th-232 | Ra-228, Ac-228, Th-228, Ra-224, Rn-220, Po-216, Pb-212, Bi-212, Tl-208 (0.36), Po-212 (0.64) |
| U-235 | Th-231, Pa-231, Th-227, Ac-227, Fr-223 (0.01), Ra-223, Rn-219, Po-215, Pb-211, Bi-211, Tl-207 |
| U-238s | Th-234, Pa-234m, Pa-234 (0.002), U-234, Th-230, Ra-226, Rn-222, Po-218, Pb-214, Bi-214, Po-214, Pb-210, Bi-210, Po-210 |
| U-238 | Th-234, Pa-234m, Pa-243 (0.002), U-234, Th-230 |
| Zr-95 | Nb-95 |

TABLE II.III. RADIONUCLIDE DECAY CONSTANTS

| Radionuclide | Decay constant (a-1) |
| --- | --- |
| Ag-110m | 1.01 100 |
| Am-241 | 1.60 10-3 |
| Ca-45 | 1.55 100 |
| Ce-141 | 7.78 100 |
| Ce-144 | 8.91 10-1 |
| Cl-36 | 2.30 10-6 |
| Cm-242 | 1.55 100 |
| Cm-243 | 2.43 10-2 |
| Cm-244 | 3.83 10-2 |
| Co-57 | 9.34 10-1 |
| Co-58 | 3.57 100 |
| Co-60 | 1.32 10-1 |
| Cr-51 | 9.13 100 |
| Cs-134 | 3.36 10-1 |
| Cs-137 | 2.31 10-2 |
| Fe-55 | 2.57 10-1 |
| Fe-59 | 5.69 100 |
| Hg-203 | 5.43 100 |
| I-125 | 4.21 100 |
| I-129 | 4.41 10-8 |
| I-131 | 3.15 101 |
| Ir-192 | 3.42 100 |
| Mn-54 | 8.11 10-1 |
| Na-22 | 2.67 10-1 |
| Nb-95 | 7.21 100 |
| Np-237 | 3.24 10-7 |
| Pb-210 | 3.11 10-2 |
| Pm-147 | 2.65 10-1 |
| Po-210 | 1.83 100 |
| Pu-238 | 7.90 10-3 |
| Pu-239 | 2.88 10-5 |
| Pu-241 | 4.81 10-2 |
| Pu-242 | 1.84 10-6 |
| Ra-224 | 6.91 101 |
| Ra-226 | 4.33 10-4 |
| Ru-103 | 6.44 100 |
| Ru-106 | 6.86 10-1 |
| S-35 | 2.89 100 |
| Sb-124 | 4.20 100 |
| Sb-125 | 2.50 10-1 |
| Se-75 | 2.11 100 |
| Sn-113 | 2.20 100 |
| Sr-85 | 3.90 100 |
| Sr-89 | 5.01 100 |
| Sr-90 | 2.38 10-2 |
| Tc-99 | 3.25 10-6 |
| Th-228 | 3.62 10-1 |
| Th-230 | 9.00 10-6 |
| Th-232 | 4.93 10-11 |
| Tl-204 | 1.83 10-1 |
| U-234 | 2.84 10-6 |
| U-235 | 9.85 10-10 |
| U-238 | 1.55 10-10 |
| Zn-65 | 1.04 100 |
| Zr-95 | 3.95 100 |

TABLE II.IV. MARINE SEDIMENT DISTRIBUTION COEFFICIENTS (m3 kg-1) AND CONCENTRATION FACTORS (m3 kg-1) FOR MARINE FISH AND SHELLFISH.

| Element | Sediment distribution coefficients (Kd) | Concentration factor (m3 kg-1) |  |
| --- | --- | --- | --- |
|  | (m3 kg-1) | Marine fish | Shellfish |
| Ag | 1 100 | 5 10-1 | 1 101 |
| Am | 2 103 | 5 10-2 | 2 101 |
| Ca | 5 10-1 | 2 10-3 | 5 10-3 |
| Ce | 2 103 | 5 10-2 | 5 100 |
| Cl | 3 10-5 | 5 10-5 | 5 10-5 |
| Cm | 2 103 | 5 10-2 | 3 101 |
| Co | 2 102 | 1 100 | 5 100 |
| Cr | 5 101 | 2 10-1 | 8 10-1 |
| Cs | 3 100 | 1 10-1 | 3 10-2 |
| Fe | 5 101 | 3 100 | 3 101 |
| Hg | 1 101 | 2 101 | 2 101 |
| I | 2 10-2 | 1 10-2 | 1 10-2 |
| Ir | 1 102 | 2 10-2 | 1 10-1 |
| Mn | 2 102 | 4 10-1 | 5 100 |
| Na | 1 10-3 | 1 10-4 | 3 10-4 |
| Nb | 5 102 | 1 10-2 | 1 100 |
| Np | 5 100 | 1 10-2 | 4 10-1 |
| Pb | 2 102 | 2 10-1 | 1 100 |
| Pm | 2 103 | 5 10-1 | 5 100 |
| Po | 2 104 | 2 100 | 5 101 |
| Pu | 1 102 | 4 10-2 | 3 100 |
| Ra | 5 100 | 5 10-1 | 1 100 |
| Ru | 3 10-1 | 2 10-3 | 2 100 |
| S | 5 10-4 | 2 10-3 | 4 10-3 |
| Sb | 1 100 | 4 10-1 | 4 10-1 |
| Se | 1 100 | 6 100 | 6 100 |
| Sn | 1 100 | 5 101 | 5 101 |
| Sr | 1 100 | 2 10-3 | 2 10-3 |
| Tc | 1 10-1 | 3 10-2 | 1 100 |
| Th | 2 103 | 6 10-1 | 1 100 |
| Tl | 2 101 | 5 100 | 5 100 |
| U | 1 100 | 1 10-3 | 3 10-2 |
| Zn | 2 101 | 1 100 | 5 101 |
| Zr | 1 103 | 2 10-2 | 5 100 |

TABLE II.V. EFFECTIVE DOSE COEFFICIENTS FOR EXTERNAL AND INTERNAL IRRADIATION

|  | External dose coefficients |  | Internal dose coefficients (Sv Bq-1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Radionuclide | Surface deposit | Material on ship | Ingestion |  | Inhalation |  |
|  | (Sv a-1 per Bq m-2) | (Sv h-1 per Bq kg-1) | Infants | Adults | Infants | Adults |
| Ag-110m | 8.5 10-8 | 5.4 10-11 | 1.4 10-8 | 2.8 10-9 | 4.1 10-8 | 1.2 10-8 |
| Am-241 | 8.9 10-10 | 0.0 | 3.7 10-7 | 2.0 10-7 | 6.9 10-5 | 4.2 10-5 |
| Ca-45 | 1.5 10-12 | 0.0 | 4.9 10-9 | 7.1 10-10 | 8.8 10-9 | 2.7 10-9 |
| Ce-141 | 2.4 10-9 | 6.3 10-14 | 5.1 10-9 | 7.1 10-10 | 1.2 10-8 | 3.8 10-9 |
| Ce-144 | 5.8 10-9 | 1.4 10-14 | 3.9 10-8 | 5.2 10-9 | 1.6 10-7 | 3.6 10-8 |
| Cl-36 | 3.6 10-10 | 0.0 | 6.3 10-9 | 9.3 10-10 | 2.6 10-8 | 7.3 10-9 |
| Cm-242 | 3.3 10-11 | 0.0 | 7.6 10-8 | 1.2 10-8 | 1.8 10-5 | 5.2 10-6 |
| Cm-243 | 4.0 10-9 | 3.9 10-13 | 3.3 10-7 | 1.5 10-7 | 6.1 10-5 | 3.1 10-5 |
| Cm-244 | 3.0 10-11 | 0.0 | 2.9 10-7 | 1.2 10-7 | 5.7 10-5 | 2.7 10-5 |
| Co-57 | 3.7 10-9 | 3.3 10-14 | 1.6 10-9 | 2.1 10-10 | 3.7 10-9 | 1.0 10-9 |
| Co-58 | 3.0 10-8 | 1.8 10-11 | 4.4 10-9 | 7.4 10-10 | 7.5 10-9 | 2.1 10-9 |
| Co-60 | 7.5 10-8 | 6.2 10-11 | 2.7 10-8 | 3.4 10-9 | 8.6 10-8 | 3.1 10-8 |
| Cr-51 | 9.8 10-10 | 2.1 10-13 | 2.3 10-10 | 3.8 10-11 | 2.1 10-10 | 3.7 10-11 |
| Cs-134 | 4.9 10-8 | 2.8 10-11 | 1.6 10-8 | 1.9 10-8 | 7.3 10-9 | 6.6 10-9 |
| Cs-137 | 1.9 10-8 | 8.0 10-12 | 1.2 10-8 | 1.3 10-8 | 5.4 10-9 | 4.6 10-9 |
| Fe-55 | 0.0 | 0.0 | 2.4 10-9 | 3.3 10-10 | 3.2 10-9 | 7.7 10-10 |
| Fe-59 | 3.6 10-8 | 3.0 10-11 | 1.3 10-8 | 1.8 10-9 | 1.3 10-8 | 4.0 10-9 |
| Hg-203 | 7.4 10-9 | 1.6 10-12 | 1.1 10-8 | 1.9 10-9 | 7.9 10-9 | 2.4 10-9 |
| I-125 | 1.4 10-9 | 0.0 | 5.7 10-8 | 1.5 10-8 | 2.3 10-8 | 5.1 10-9 |
| I-129 | 8.3 10-10 | 0.0 | 2.2 10-7 | 1.1 10-7 | 8.6 10-8 | 3.6 10-8 |
| I-131 | 1.2 10-8 | 4.5 10-12 | 1.8 10-7 | 2.2 10-8 | 7.2 10-8 | 7.4 10-9 |
| Ir-192 | 2.6 10-8 | 8.3 10-12 | 8.7 10-9 | 1.4 10-9 | 1.8 10-8 | 5.2 10-9 |
| Mn-54 | 2.6 10-8 | 1.5 10-11 | 3.1 10-9 | 7.1 10-10 | 6.2 10-9 | 1.5 10-9 |
| Na-22 | 6.7 10-8 | 5.2 10-11 | 1.5 10-8 | 3.2 10-9 | 7.3 10-9 | 1.3 10-9 |
| Nb-95 | 2.4 10-8 | 1.5 10-11 | 3.2 10-9 | 5.8 10-10 | 5.9 10-9 | 1.8 10-9 |
| Np-237 | 7.2 10-9 | 1.2 10-12 | 2.1 10-7 | 1.1 10-7 | 4.0 10-5 | 2.3 10-5 |
| Pb-210 | 1.2 10-9 | 1.6 10-16 | 3.6 10-6 | 6.9 10-7 | 3.7 10-6 | 1.1 10-6 |
| Pm-147 | 1.1 10-12 | 8.5 10-20 | 1.9 10-9 | 2.6 10-10 | 1.8 10-8 | 5.0 10-9 |
| Po-210 | 2.6 10-13 | 1.6 10-16 | 8.8 10-6 | 1.2 10-6 | 4.8 10-6 | 6.1 10-7 |
| Pu-238 | 2.9 10-11 | 0.0 | 4.0 10-7 | 2.3 10-7 | 7.4 10-5 | 4.6 10-5 |
| Pu-239 | 1.3 10-11 | 1.4 10-18 | 4.2 10-7 | 2.5 10-7 | 7.7 10-5 | 5.0 10-5 |
| Pu-241 | 2.7 10-11 | 0.0 | 1.7 10-8 | 1.1 10-8 | 3.0 10-6 | 2.1 10-6 |
| Pu-242 | 2.3 10-11 | 0.0 | 4.0 10-7 | 2.4 10-7 | 7.3 10-5 | 4.8 10-5 |
| Ra-224 | 4.7 10-8 | 4.5 10-11 | 6.6 10-7 | 6.5 10-8 | 8.2 10-6 | 3.0 10-6 |
| Ra-226 | 5.7 10-8 | 3.9 10-11 | 9.6 10-7 | 2.8 10-7 | 1.1 10-5 | 3.5 10-6 |
| Ru-103 | 1.5 10-8 | 6.6 10-12 | 4.6 10-9 | 7.3 10-10 | 8.4 10-9 | 2.4 10-9 |
| Ru-106 | 1.1 10-8 | 3.2 10-12 | 4.9 10-8 | 7.0 10-9 | 1.1 10-7 | 2.8 10-8 |
| S-35 | 5.5 10-13 | 0.0 | 5.4 10-9 | 7.7 10-10 | 4.5 10-9 | 1.4 10-9 |
| Sb-124 | 5.6 10-8 | 3.9 10-11 | 1.6 10-8 | 2.5 10-9 | 2.4 10-8 | 6.4 10-9 |
| Sb-125 | 1.4 10-8 | 5.4 10-12 | 6.1 10-9 | 1.1 10-9 | 1.6 10-8 | 4.8 10-9 |
| Se-75 | 1.2 10-8 | 2.4 10-12 | 1.3 10-8 | 2.6 10-9 | 6.0 10-9 | 1.3 10-9 |
| Sn-113 | 8.8 10-9 | 2.8 10-12 | 5.0 10-9 | 7.3 10-10 | 1.0 10-8 | 2.7 10-9 |
| Sr-85 | 1.6 10-8 | 6.6 10-12 | 3.1 10-9 | 5.6 10-10 | 3.7 10-9 | 8.1 10-10 |
| Sr-89 | 2.2 10-9 | 3.3 10-15 | 1.8 10-8 | 2.6 10-9 | 3.0 10-8 | 7.9 10-9 |
| Sr-90 | 3.5 10-9 | 0.0 | 7.3 10-8 | 2.8 10-8 | 5.2 10-8 | 2.4 10-8 |
| Tc-99 | 2.5 10-12 | 0.0 | 4.8 10-9 | 6.4 10-10 | 2.5 10-9 | 2.9 10-10 |
| Th-228 | 4.7 10-8 | 4.5 10-11 | 7.2 10-8 | 7.2 10-8 | 1.1 10-4 | 3.2 10-5 |
| Th-230 | 5.7 10-8 | 3.9 10-11 | 2.1 10-7 | 2.1 10-7 | 7.4 10-5 | 4.3 10-5 |
| Th-232 | 7.8 10-8 | 6.5 10-11 | 4.5 10-7 | 2.3 10-7 | 5.0 10-5 | 2.5 10-5 |
| Tl-204 | 3.5 10-10 | 5.0 10-19 | 8.5 10-9 | 1.2 10-9 | 3.3 10-9 | 3.9 10-10 |
| U-234 | 2.6 10-11 | 1.2 10-18 | 1.3 10-7 | 4.9 10-8 | 2.9 10-5 | 9.4 10-6 |
| U-235 | 2.2 10-8 | 3.5 10-12 | 1.3 10-7 | 4.7 10-8 | 2.6 10-5 | 8.5 10-6 |
| U-238 | 3.9 10-9 | 3.0 10-13 | 1.2 10-7 | 4.5 10-8 | 2.5 10-5 | 8.0 10-6 |
| U-238s | 6.1 10-8 | 3.9 10-11 | 1.2 10-7 | 4.5 10-8 | 2.5 10-5 | 8.0 10-6 |
| Zn-65 | 1.8 10-8 | 1.1 10-11 | 1.6 10-8 | 3.9 10-9 | 1.0 10-8 | 2.2 10-9 |
| Zr-95 | 4.7 10-8 | 3.0 10-11 | 5.6 10-9 | 9.5 10-10 | 1.6 10-8 | 4.8 10-9 |

TABLE II.VI. DEFAULT VALUES FOR PARAMETERS USED IN THE CALCULATION OF RADIONUCLIDE CONCENTRATIONS IN ENVIRONMENTAL MATERIALS

| Parameter | Symbol | Value |
| --- | --- | --- |
| Mass of candidate material dumped (kg a-1) | M | 1 109 |
| Volume of the box (m3) | V | 2 109 |
| Flux of water through coastal region (m3 a-1) | F | 4 1010 |
| Depth of water column (m) | D | 2 101 |
| Thickness of boundary sediment layer in box (m) | LB | 1 10-2 |
| Effective sediment thickness (m) | dS | 1 10-1 |
| Bulk sediment and waste density (kg m-3) | ñS, ñB | 1.5 103 |
| Suspended sediment concentration (kg m-3) | S | 3 10-3 |

TABLE II.VII. DEFAULT VALUES OF PARAMETERS USED IN THE CALCULATION OF INDIVIDUAL DOSES

| Parameter | Symbol | Adult | Infant (1 to 2 years) |
| --- | --- | --- | --- |
| Crew |  |  |  |
| Occupancy on board of ship (h a-1) | tcrew | 1 103 | – |
| Breathing rate (m3 a-1) | Rinh, crew | 1.05 104 | – |
| Dust loading on board ship (kg m-3) | DLship | 2.5 10-10 | – |
| Inadvertent ingestion rate of material (kg a-1) | Hdust | 1 10-2 | – |
| Members of the public |  |  |  |
| Occupancy on sediments on shore (h a-1) | tpublic | 1.6 103 | 1 103 |
| Breathing rate (m3 a-1) | Rinh, public | 8.4 103 | 1.4 103 |
| Dust loading on shore (kg m-3) | DLshore | 2.5 10-10 | 2.5 10-10 |
| Seaspray concentration in air (kg m-3) | Cspray | 1 10-2 | 1 10-2 |
| Ingestion rate of fish (kg a-1) | HB(fish) | 5 101 | 2.5 101 |
| Ingestion rate of shellfish (kg a-1) | HB(shellfish) | 1.5 101 | 0 |

TABLE II.VIII. DEFAULT VALUES FOR PARAMETERS USED IN THE CALCULATION OF COLLECTIVE DOSES

| Parameter | Symbol | Value |
| --- | --- | --- |
| Number of dumping sites | Nsites | 1 101 |
| Number of ships for dumping site | Nship | 1 100 |
| Number of crew members on a ship | Ncrew | 1 101 |
| Annual fish catch in the area of a single site (kg a-1) | NB(fish) | 1 106 |
| Annual shellfish catch in the area of a single site (kg a-1) | NB(shellfish) | 4 105 |
| Fraction of fish caught in the area consumed by man | fB(fish) | 5 10-1 |
| Fraction of shellfish caught in the area consumed by man | fB(shellfish) | 3.5 10-1 |
| Annual collective shore occupancy per unit length (man h a-1 m-1) | Ocoll,shor | 5 101 |
| Coastline length for one site (m) | Lshore | 1 104 |

References

| [II.1] | INTERNATIONAL COMMISSION ON RADIOLOGICAL PROTECTION, Radionuclide Transformation, Energy and Intensity of emission, ICRP Publication 38, Pergamon Press, Oxford (1993). |
| --- | --- |
| [II.2] | INTERNATIONAL ATOMIC ENERGY AGENCY, Sediment Kds and Concentration Factors for Radionuclides in the Marine Environment, IAEA Technical Report Series No. 247, Vienna (1985). |
| [II.3] | Program MicroShield, Version 5.01, 1995-96, Grove Engineering, USA |
| [II.4] | EKERMAN, K.F, RYMAN, J. C., External Exposure to Radionuclides in Air, Water and Soil, Federal Guidance Report No. 12, US Environmental Protection Agency, Washington D.C. (1993). |
| [II.5] | INTERNATIONAL ATOMIC ENERGY AGENCY, International Basic Safety Standards for Protection against Ionizing Radiation and for the Safety of Radiation Sources (jointly sponsored by FAO, IAEA, ILO, OECD/NEA, PAHO and WHO), Safety Series No. 115, IAEA, Vienna (1996). |
| [II.6] | SIMMONDS, J.R., LAWSON, G., MAYALL, A., Methodology for Assessing the Radiological Consequences of Routine Releases of Radionuclides to the Environment, EUR 15 760, European Commission, Luxembourg (1995). |
| [II.7] | INTERNATIONAL ATOMIC ENERGY AGENCY, Definition and Recommendations for the Convention on the Prevention of Marine Pollution by Dumping of Wastes and other Matter, 1972, 1986 Edition, Safety Series No. 78, IAEA, Vienna (1986). |
| [II.8] | INTERNATIONAL ATOMIC ENERGY AGENCY, Generic Models for Use in Assessing the Impact of Discharges of Radioactive Substances to the Environment, IAEA Safety Reports Series No. 19, Vienna (2001). |
| [II.9] | COMMISSION OF EUROPEAN COMMUNITIES, Principles and methods for Establishing Concentrations and Quantities (Exemption Values) below which Reporting is not Required in the European Directive, Radiation Protection 65, CEC, Luxembourg (1993). |
| [II.10] | INTERNATIONAL ATOMIC ENERGY AGENCY, Clearance of Materials Resulting from the Use of Radionuclides in Medicine, Industry and Research, IAEA-TECDOC-1000, Vienna (1998). |
| [II.11] | INTERNATIONAL COMMISSION ON RADIOLOGICAL PROTECTION, Human Respiratory Model for Radiological Protection, ICRP Publication 66, Pergamon Press, Oxford (1993). |
| [II.12] | COMMISSION OF THE EUROPEAN COMMUNITIES, The radiological exposure of the population of the European Community from radioactivity in North European marine waters, Project Marina, Radiation Protection 47, CEC, Luxembourg (1990) |

aNNEX I
(From the Contracting Parties to the LC)
GUIDELINES FOR THE APPLICATION OF THE DE MINIMIS CONCEPT UNDER THE LONDON CONVENTION1

1	INTRODUCTION

1.1	The London Convention prohibits the disposal at sea of radioactive wastes and other radioactive matter. However, all materials, including natural and inert materials, contain natural radionuclides and are frequently contaminated with artificial radionuclides from such anthropogenic sources as fallout due to past atmospheric nuclear testing. Therefore, the Contracting Parties to the London Convention recognized the need to develop definitions and guidelines whereby candidate materials (those wastes or other matter not otherwise prohibited from disposal at sea in accordance with Annex I to the Convention) containing de minimis levels of radionuclides could be disposed of pursuant to the provisions of this Convention. The full text of Annex I to the London Convention, as amended in 1993, is attached in an appendix to these Guidelines.

1.2	The concept of “de minimis” for radioactive substances was initially discussed in 1976 at the First Consultative Meeting of Contracting Parties to the London Convention. Since that time, the International Atomic Energy Agency (IAEA) has prepared several reports on the subject, all of which reflect contemporary development of the concept at the time of publication. Parallel to progress in the field of radiation protection, there have been developments in the framework of the Convention itself. In 1993, the Annexes I and II to the London Convention were amended to prohibit the dumping at sea of radioactive wastes or other radioactive matter. At the 19th Consultative Meeting in 1997, Contracting Parties agreed to request the IAEA to develop further the concept of de minimis and, in particular, to "provide guidance for making judgements on whether materials planned to be dumped could be exempted from radiological control or whether a specific assessment was needed" (LC 19/10, paragraph 6.31). This paragraph continues: “The IAEA would then further be requested to provide guidance to national authorities responsible for conducting specific assessments.”

1.3	In 1998, the IAEA presented its advice on de minimis in a draft document (LC 20/7) to the Twentieth Consultative Meeting of Contracting Parties. The Consultative Meeting requested the IAEA to revise this document based on comments made by Contracting Parties. The Consultative Meeting agreed that in further developing the concept of de minimis, the following issues should be considered in detail (LC 20/14, paragraph 7.9):

".1	to ensure that the de minimis concept applies only to those wastes or other matter not otherwise prohibited from disposal under the Convention;

.2	the protection of the marine environment including human health, flora and fauna of the marine environment as well as legitimate uses of the sea; and

.3	the need for practical and uniform guidance to national authorities responsible for authorizing sea disposal activities."

1.4	The revised text of the IAEA report has been distributed as IAEA-TECDOC-1068 entitled: "Application of Radiological Exclusion and Exemption Principles to Sea Disposal".

1.5	The following text provides specific guidance2 regarding the definition and application of the de minimis concept only to candidate materials. Section 2 of this guidance reproduces relevant sections of the work of the International Atomic Energy Agency (IAEA) to develop a concept of de minimis for the purposes of the London Convention, set forth in IAEA-TECDOC-1068, March 1999.

1.6	Section 3 of this guidance sets forth a clarification of this IAEA advice, with relevant examples of how the IAEA would apply the “de minimis” concept under the London Convention.

1.7	Section 4 of this guidance sets forth a stepwise evaluation procedure for use under the London Convention for determining whether candidate materials can be treated as de minimis3.

2	THE IAEA ADVICE ON “ ‘DE MINIMIS’ (EXEMPTION5) CRITERIA FOR CANDIDATE MATERIALS FOR SEA DUMPING UNDER THE LONDON CONVENTION”

2.1	In IAEA-TECDOC-1068, page 10, the first paragraph under this heading, deals with the criteria for exemption without further consideration as follows:

" Materials eligible for consideration for dumping at sea under the London Convention that can be assigned as ‘de minimis’ (exempt5) without further consideration from the perspective of their radionuclide content are therefore those containing only:

(1)	natural radionuclides in environmental and raw materials, unless there is concern on the part of the national regulatory authority that the radiation field would be substantially modified;

(2)	radionuclides in materials derived from activities involving some modification of the natural radionuclide composition that has been considered by the national regulatory authority, and deemed not to warrant radiological control, having taken proper account of the marine environmental and other conditions relevant to the disposal, re-use and relocation of such materials;

(3)	widely-distributed radionuclides resulting from global fallout from nuclear weapons tests, satellite burnup in the stratosphere, and accidents, that have led to widespread dispersion of radionuclides that are deemed by the national regulatory authority not to warrant intervention; and

(4)	radionuclides arising from sources and practices that have been exempted or cleared nationally from radiological control, pursuant to the application of the international criteria for exemption and clearance, where proper account has been taken of the marine environmental and other conditions relevant to potential disposal, re-use and relocation of such materials."

Footnote 5 of IAEA-TECDOC-1068 reads as follows:

"The term 'exemption for the purposes of the London Convention' is taken to mean ‘de minimis’, that is, it includes both the radiological concepts of exclusion and exemption."

2.2	The concluding text of the final section of IAEA-TECDOC-1068 deals with materials that cannot be exempted without further consideration for the purposes of the London Convention (note that footnote 5 of IAEA-TECDOC-1068 applies equally here also). This text reads as follows:

"Candidate materials that cannot be exempted5 without further consideration may then be subjected to a specific assessment to determine if they still qualify as exempt5 for the purposes of the London Convention. Such specific assessments would need to be carried out by national regulatory authorities using the radiological criteria for exemption set out in Section 3 (of  IAEA-TECDOC-1068). It should be noted in this context that assessments are required for proposed dumping activities in relation to other characteristics and properties of Candidate Materials than radioactivity pursuant to the provisions of Article IV and Annex III of the Convention. The specific assessment required to consider further exemption5 of materials for determining if they can be treated as 'non-radioactive', would include an evaluation of the radiological implications for human health and the environment (see Section 3 of IAEA-TECDOC-1068).

In cases where Candidate Materials are either contaminated by, or derived from, authorized or unauthorized releases, each situation would have to be reviewed in its specific context. The need for intervention may also be a relevant consideration in certain cases."

2.3	The principles and criteria for exemption are described in Section 3 of IAEA-TECDOC-1068 and summarized in the concluding section of that report as follows:

"A practice, or source within a practice, may be exempted without further consideration provided that the following radiological criteria are met in all feasible situations:

(a)	the effective dose expected to be incurred by any member of the public due to the exempted practice or source is of the order of 10 mSv or less in a year; and

(b)	either the collective dose committed by one year of performance of the practice is not more than about 1 man.Sv or an assessment for the optimization of protection shows that exemption is the optimum option."

3	MEANING AND IMPLICATIONS OF THE IAEA ADVICE

3.1	The IAEA advice presented in Section 2 above may require some clarification for an audience unfamiliar with the terminology used in the field of radiological protection. Such explanation is provided here as a basis for developing appropriate definitions and guidelines under the London Convention. It should be noted that all reference to the 'national regulatory authority' in the IAEA's advice quoted in Section 2 above refers to the national radiation protection authority.

3.2	All materials contain natural radionuclides. Sometimes human activities can result in changes to their concentrations, thus potentially increasing radiation doses. If this occurs, an activity could be subjected to radiological control. Previous human activities, particularly nuclear weapons testing in the atmosphere, have also introduced new radionuclides to the environment and slightly enhanced the overall concentrations of radionuclides.

3.3	The current system of radiological protection is based entirely on the protection of human health. This has been developed over many decades and there are now internationally accepted guidelines and standards for national radiation protection authorities. There is currently no internationally-accepted basis for the protection of the environment, including flora and fauna, from the effects of radiation. Accordingly, the IAEA advice with regard to the de minimis issue is based on the protection of human health.

3.4	The IAEA advice provides for two distinct categories in relation to the de minimis question:

- first, cases in which the radionuclide constituents of a candidate material fall within the provisions of Section 2.1 above and can be automatically (i.e., without further consideration) defined as de minimis under the London Convention; and

- second, cases in which a specific assessment is required to determine whether the candidate materials are de minimis or not (see Section 2.2 above).

Automatic Exemption Criteria

3.5	The paragraph of the IAEA advice quoted in Section 2.1 above deals with materials eligible for dumping at sea under the London Convention without further consideration from the perspective of radiological protection. The provisions of this paragraph correspond to 'automatic' or 'default' assignment of de minimis, that is exempted from any concerns regarding the radioactive content of the candidate materials (i.e., materials eligible for dumping at sea) from the perspective of radiological protection. The following subparagraphs need to be considered before this assignment can be made. Each of the sub-paragraphs specifies classes of constituent radionuclides that according to the IAEA advice can be considered as de minimis for the purposes of the London Convention.

.1	Sub-paragraph 2.1(1) specifies that natural radionuclides in naturally-occurring materials are de minimis unless the national radiation protection authority has registered concern, from radiological perspectives, about the radiation field being substantially increased. In most cases, movement of such materials from one location in the marine environment to another presents relatively minor modification of the prevailing radiation fields in both the original and destination (dump)site. Thus, such cases are not regarded as of concern. The kind of natural materials over which such concerns might be registered by the national radiation protection authority could include the deliberate relocation of natural materials that are naturally enriched in naturally-occurring radionuclides (e.g., monazite sands) to an environment in which the natural radiation field is low. Unless such concerns have been raised, natural radionuclides in unmodified natural environmental materials can be assigned as de minimis and automatically exempted without further consideration or assessment. In instances in which concerns had been expressed by the national radiation protection authority, a specific assessment would be required to determine their suitability for de minimis assignment.

.2	Sub-paragraph 2.1(2) deals with natural radionuclides associated with materials derived from human activities that have resulted in some redistribution of natural radionuclides such that the concentrations in otherwise natural materials may have been changed. For some countries such activities could include application to soil of phosphate fertilizer. For other countries, such activities could include the processing of minerals, e.g., fertilizer production from phosphate-rich geological materials. In this case the distribution of natural radionuclides (e.g., phosphorus-31, uranium and thorium decay radionuclides) is altered in the process. It should be noted that some countries regard processing of such materials as a regulated practice, not one from which the exposures are excluded. Wastes from the process may be discharged into the aquatic environment and incorporated into sediments that may need to be dredged. The national regulatory authority may have evaluated this practice and made a judgement regarding its inclusion or exclusion from regulatory control. If a decision on the exclusion of exposures associated with the practice has been made based on evaluations of the entire practice taking into account marine pathways of exposure, the decision translates into automatic exemption for the purposes of the London Convention. No further account would then need to be taken of the radioactive constituents or radiological effects of materials derived from or affected by that practice. If such is not the case, a specific assessment would be needed to determine if the candidate material could be assigned as de minimis or not.

.3 	Sub-paragraph 2.1(3) deals predominantly with artificial radionuclides stemming from nuclear weapons testing in the atmosphere, from satellite radiothermal power unit burnup in the stratosphere and nuclear accidents. Atmospheric fallout is a collective term but primarily comprises radionuclides, particularly fission products, from nuclear weapons tests that were conducted in the atmosphere in the period 1945 - 1981. These have been augmented by specific radionuclides such as plutonium-238 from thermonuclear generators that have burnt up during re-entry to the atmosphere and the more widely-dispersed radionuclides (i.e., radionuclides distributed globally rather than locally or regionally) from nuclear accidents. Global fallout results in the contamination of soils and sediments throughout the surface of the Earth. The relocation of aquatic sediments contaminated by global fallout over relatively small distances, as is effected by dredging and dumping activities, for example, neither significantly alters the distribution of such radionuclides nor significantly changes their environmental concentrations. In cases where contamination by radionuclides from such sources has been judged by the national radiation protection authority not to warrant intervention to reduce associated human exposures, the materials so contaminated can be assigned as de minimis and exempted without further consideration or assessment. In other circumstances a specific assessment would be needed to determine if the candidate material could be assigned as de minimis or not.

.4	Sub-paragraph 2.1(4) deals with radionuclides derived from sources and practices that have been exempted or cleared from radiological control by the national radiation protection authority, consistent with applicable international criteria. If the national radiation protection authority has exempted a practice, or cleared from radiological control a previously regulated practice, based on an assessment of the practice and any disposal, re-use and relocation of materials from the practice, including taking account of marine environmental exposure pathways, the radionuclides derived from that practice can be assigned as de minimis and automatically exempted from radiological concerns without further consideration. In instances in which these conditions are not fulfilled, a specific assessment would be needed to determine if the candidate materials could be assigned as de minimis or not.

3.6	Assignment of materials as de minimis based on the above criteria merely relieves the permitting authority of any requirement to further consider the radioactive properties of such materials and the radiological consequences of their disposal. The other characteristics and properties of candidate materials still remain to be assessed in the context of the suitability of the candidate materials for disposal at sea under the London Convention through application of the provisions of Article IV and Annexes I and III of the Convention and their suitability for disposal at sea through application of the Guidelines for the Assessment of Wastes or Other Matter, that May be Considered for Dumping.

Specific Assessments

3.7	The two paragraphs of the IAEA advice quoted in Section 2.2 above deal with the situation that applies if the assignment of de minimis cannot be made automatically (i.e., without further consideration). The first paragraph defines the nature of a 'specific assessment' that would be required to be undertaken by the national regulatory authority using the international radiological criteria for exemption as quoted in Section 2.3 above and other considerations relating to the radiological implications for the environment.

3.8	The second paragraph quoted in Section 2.2 above deals specifically with contamination of candidate materials by authorized discharges or unauthorized releases of radionuclides to the environment. Authorized discharges are those from regulated practices such as nuclear power reactor operations and nuclear fuel reprocessing. Unauthorized releases are either illicit or unintentional. In such cases, a specific assessment using relevant international radiological criteria for exemption would also be needed regarding the suitability of the material for assignment as de minimis.

3.9	Again, as in the case of de minimis assignment without further consideration (see paragraph 3.6 above), irrespective of any determination that a candidate material is de minimis from the perspective of its radionuclide content or radioactive properties, that material would still be subject to the provisions of the London Convention, in particular, Article IV and Annexes I and III and its suitability for disposal at sea would require to be assessed through the application of the Guidelines for the Assessment of Wastes or Other Matter, that May be Considered for Dumping.

4	EVALUATION PROCEDURE FOR DEFINING DE MINIMIS

Introduction

4.1	This Section describes the application of the IAEA de minimis definition when assessing candidate materials under the London Convention. The intent is to assess candidate materials to determine if they contain de minimis levels of radioactivity or if a specific assessment is required4 . This evaluation procedure is intended to be implemented through judgements based on available information regarding the provenance of candidate materials and sediments in the receiving marine environment, specifically at the dump-site. The questions posed at each of the first five steps are designed to be answered without the need for direct measurements of radionuclides in either the candidate material or the marine environment. Indeed, had such a requirement been a prerequisite to the first five steps of this procedure, it would run entirely counter to the intent and interpretation of de minimis.

4.2	In cases when there is insufficient existing information on which to base such judgements a specific assessment would be required.

4.3	The field of radiological protection is evolving. Parties should take all relevant advances into account when applying the guidelines. For example, criteria for evaluating the impacts of radioactivity on the marine environment are advancing, and should, when available and as relevant, be expeditiously utilized.

STEPWISE EVALUATION PROCEDURE

STEP 1:	CANDIDATE MATERIALS

Decision Criteria:	Candidate materials are those wastes or other matter not otherwise prohibited from disposal by Annex I of the London Convention.

1	Are the proposed materials eligible for dumping under the provisions of the London Convention?

2	If NO, the material is not allowed to be dumped and no further consideration is warranted.

3	If YES, go to Step 2.

STEP 2:	INITIAL SCREEN FOR SOURCES OF CONTAMINATION

Decision Criteria:	Virtually all candidate materials are likely to have some level of radioactivity due to natural radionuclides at background levels and artificial radionuclides derived from global fallout. Global atmospheric fallout is a collective term but primarily comprises radionuclides, particularly fission products, from nuclear weapons tests that were conducted in the atmosphere in the period 1945 - 1981. These have been augmented by specific radionuclides such as plutonium-238 from thermonuclear generators that have burnt up during re-entry into the atmosphere. If candidate materials for dumping at sea contain only such natural radionuclides at locally prevailing background levels in the vicinity of the proposed dump-site and artificial radionuclides from global fallout, they can be immediately assigned as de minimis.

If the result of the initial screen leads to a conclusion that there is no reason to believe that the candidate material is a modified natural material or contaminated from other sources,  the material is considered de minimis, unless there is concern on the part of the regulatory authority that the radiation field in the vicinity of the dump-site would be substantially modified. (this latter situation is dealt with at Step 4 of this procedure).

1	Is there reason to believe that the candidate material contains anything other than unmodified natural radionuclides at background comparable with that in the receiving environment and artificial radionuclides derived from global fallout?

2	If NO, the materials are de minimis.

3	If YES, go to Step 3.

STEP 3:	ASSESSMENT OF ADDITIONAL CAUSES/SOURCES

Decision Criteria:	If there are additional radionuclides in the candidate material, it is important to discriminate between the causes/sources of the presence of these additional radionuclides. Increases in the presence of radionuclides at the dump-site could result from two causes: (1) differences in the concentrations of natural radionuclides in the candidate material and in sediments at the dump-site, and; (2) human activities that increase the concentrations of natural radionuclides in candidate materials. The permitting authority should address both possibilities before determining if levels of radioactivity in the materials are de minimis. The first of these causes is addressed in Step 4 of this procedure. The second is considered in Step 5.

This Step is intended to determine the nature of causes/sources responsible for any additional radioactivity in the candidate material.

1	What are the likely additional causes/sources contributing to the radioactivity in the materials?

2	If only unmodified natural causes/sources, go to Step 4.

3	If only anthropogenic causes/sources, go to Step 5.

4	If both anthropogenic and natural causes/sources, go to Step 5

STEP 4:	NATURAL CAUSES / SOURCES

Decision Criteria:	Candidate materials of natural origin unmodified by human activities are de minimis, unless the national permitting authority is concerned that the radioactivity would be substantially increased at the dump-site as a result of the dumping action.

This Step addresses the issue of whether the radiation field at the dump-site will be substantially altered by dumping of a candidate material containing natural radionuclides at unusual concentrations as a result of natural processes.

Information pertinent to this determination would include any assessment conducted by the national radiation protection authority.

1	If the material were to be dumped, would it substantially increase radioactivity at the dump-site?

2	If NO, the materials are de minimis.

3	If YES, go to Step 6.

STEP 5:	ANTHROPOGENIC CAUSES/SOURCES

Decision Criteria:	For candidate materials containing artificial radionuclides (other than from global fallout that is referred to in Step 2) and/or altered natural radionuclides stemming from human activities, the national permitting authority should consider previous decisions and action taken by the national radiation protection authority. The national permitting authority should assess whether the human activity contributing to the radioactivity in the candidate material is from an activity that has been exempted or cleared or one from which radiation exposures have been excluded by the national radiation protection authority based upon international radiological criteria. The pertinent question in such cases is whether the decisions on exclusion, exemption, or clearance were made considering marine environmental pathways of exposure and whether these are suitable to an assessment of the proposed dumping operation. If this is the case, the materials are de minimis.

1	Were the likely anthropogenic causes/sources part of exempted or cleared practices or excluded exposures?

2	If NO, go to Step 6.

3	If YES, were the marine environmental exposure pathways considered by the national radiation protection authority and are these suitable to an assessment of the proposed dumping operation?

3.1	If YES, the materials are de minimis.

3.2	If NO, go to Step 6.

STEP 6:	SPECIFIC ASSESSMENT

Materials not determined to be de minimis through the evaluation in Steps 1 - 5 above could also be determined to be de minimis by the application of a specific assessment. The foregoing Steps of this evaluation procedure lead to initial perspectives on the nature and requirements of a specific assessment as follows.

A specific assessment should provide an evaluation of the potential adverse impacts to the marine environment including effects upon human health and to flora and fauna, and to other legitimate uses of the sea. The nature and extent of the specific assessment should be determined in accordance with existing knowledge of the sources and likely extent of any radioactive contamination of the candidate material. For example, candidate dredged material containing only minor amounts of radionuclides may not need to be subjected to an unnecessarily detailed or unnecessarily complex assessment process. International radiological exemption criteria based on the protection of human health should be used for part of this assessment. Until complementary international radiological criteria for the protection of flora and fauna are developed, permitting authorities should use appropriate scientific information and a precautionary approach (as provided for in resolution LDC.44(14)) in conducting assessments of the potential impacts on the marine environment.

Guidance for the conduct of specific assessments is being developed by the IAEA. In the interim, when a Contracting Party authorizes the dumping of material in the area of the high seas, on the basis of a specific assessment, that Contracting Party should include in its reports to the Secretariat the assessment criteria used.

Application of the Waste Assessment Guidance

Candidate materials that are determined to be de minimis through the evaluation in Steps 1 - 6 above must then be evaluated through application of the generic Guidelines for the Assessment of Wastes or Other Matter that May be Considered for Dumping, and/or the waste-specific guidance developed thereunder. No further evaluation of the radionuclide content of the candidate materials that are de minimis is needed.

The present Guidelines have been developed on the basis of existing scientific knowledge of the radiation protection considerations and on a knowledge of current technology. Scientific work and technical development is, however, proceeding and consequently these Guidelines should be kept under review as the results of further research and investigation become available.

ANNEX II
EXAMPLE CALCULATIONS

Summary of Equations for Screening Assessment Eind, crewj=Dind, crewj⋅Cj (Eq. 1) Eind, publicj=Dind, publicj⋅Cj (Eq. 2) Eind,crew=jNEind, crewj10mSva−1 (Eq. 3) Eind, public=M1´109⋅jNEind, publicj10mSva−1 (Eq. 4) Ecoll, crewj=Dcoll, crewj⋅Cj (Eq. 5) Ecoll, publicj=Dcoll, publicj⋅ Cj (Eq. 6) Ecoll, crew=jNEcoll, crewj (Eq. 7) Ecoll, public= M1´109⋅jNEcoll, publicj (Eq. 8) Ecoll, tot=Ecoll, crew+Ecoll, public≤1 man/Sv a-1 (Eq. 9) Dose per Unit Activity Concentration Factors: Use the factors for individual and collective dose from Tables 3.2 and 3.3.

I.	EXAMPLE CALCULATION FOR ONE RADIONUCLIDE

This example calculation might refer to dredged material which originates from a location with sands with elevated levels of radio nuclides of natural origin (such as monazite sands).

- Determine:

- Radionuclides present and their activity concentrations;

- Mass of material for disposal.

- Results:

- Radionuclide: 137Cs with an activity concentration of 300 Bq kg-1 dry weight;

- Mass of material for disposal: 3 ´ 109 kg dry weight.

- Individual dose:

- Apply the Procedure outlined in Section 3 together with Equations (1) to (4) provided in the box above

Thus,	Eind, crew=8.1´10−3mSv⋅kga⋅Bq´300Bqkg=2.4mSv a-1

and,	Eind, public=3´109kga-11´109kga-1´3.1´10−3mSv⋅kga⋅Bq´300Bqkg=2.8mSv a-1

- Compare results with the individual dose criterion of 10 mSv a-1

- Both Eind,crew and Eind,public (i.e. dose for crew members and members of the public) are less than 10 µSv a-1.

- Collective dose:

- Because the individual dose criterion is fulfilled, it is necessary to consider whether the condition related to collective dose is also satisfied.

- Use Equations (5) to (9) from the box above:

Thus,	Ecoll,crew=8.1´10−6manSv⋅kga⋅Bq´300Bqkg=2.4´10−4manSva−1

and,	Ecoll,public=3´109kga−11´109kga−1´6.2´10−5manSv⋅kga⋅Bq´300Bqkg=5.6´10−2manSva−1

Ecoll=2.4´10−4manSva−1+5.6´10−2manSva−1=5.6´10−2manSva−1

- Compare results with condition for collective dose of 1 manSv a-1:

- The collective dose criterion is fulfilled.

- Conclusion:

- The candidate material satisfies both the individual dose criterion and the additional condition for collective dose and can therefore be considered as de minimis under the London Convention.

II.	EXAMPLE CALCULATION WITH MULTIPLE RADIONUCLIDES

This example calculation might refer to sewage sludge which contains artificial and natural radionuclides.

- Determine:

- Radionuclides present and their activity concentrations;

- Mass of material for disposal.

- Results:

- Radionuclides: 131I, 238U and 99Tc with activity concentrations of 510, 40 and 1000 Bq kg-1 dry weight respectively;

- Mass of material for disposal: 1 ´ 109 kg a-1 dry weight.

- Individual dose:

- Apply the Procedure outlined in Section 3 together with Equations (1) to (4) provided in the box above. As no activity concentrations are given for 226Ra, the values for 238U are those given in Tables 3.1 and 3.2 for U-238s. The result is:

Thus,	Eind,crew=4.8´10−3mSv⋅kga⋅Bq´510Bqkg+6.4´10−2mSv⋅kga⋅Bq´40Bqkg+6.4´10−6mSv⋅kga⋅Bq´1000Bqkg5.0mSva−1

Eind,public=1´109kga−11´109kga−1´4.4´10−4mSv⋅kga⋅Bq´510Bqkg+8.5´10−2mSv⋅kga⋅Bq´40Bqkg

- Compare results with the individual dose criterion of 10 µSv a-1.

- Both Eind,crew and Eind,public (i.e. dose for crew members and members of the public) are less than 10 µSv a-1.

- Collective dose:

- Because the individual dose criterion is fulfilled, it is necessary to consider whether the condition related to collective dose is also satisfied.

- Use Equations (5) to (9) from the box above:

Ecoll,crew=4.8´10−7manSv⋅kga⋅Bq´510Bqkg+6.4´10−6manSv⋅kga⋅Bq´40Bqkg+6.4´10−10manSv⋅kga⋅Bq´1000Bqkg5.0´10−4manSva−1

Ecoll,public=1´109kga−11´109kga−1´1.3´10−5manSv⋅kga⋅Bq´510Bqkg+6.3´10−3manSv⋅kga⋅Bq´40Bqkg

Ecoll,tot=5.0´10−4manSva−1+2.8´10−1manSva−1=2.8´10−1manSva−1

- Compare results with the collective dose criterion of 1 manSv a-1.

- The collective dose criterion is fulfilled.

- Conclusion:

- The candidate material satisfies both the individual dose criterion and the additional condition for collective dose and can therefore be considered as de minimis under the London Convention.

- III.	EXAMPLE CALCULATION WITH MULTIPLE RADIONUCLIDES AND HIGHER RADIONUCLIDE CONCENTRATIONS

This example calculation might refer to candidate material which contains natural radionuclides as a result of a practice which has been neither exempted nor cleared.

- Determine:

- Radionuclides present and their activity concentrations;

- Mass of material for disposal.

- Results:

- Radionuclides: 226Ra, 228Th and 238U with activity concentrations of 170, 100 and 140 Bq/kg dry weight respectively

- Mass of material for disposal: 5 ´ 108 kg a-1 dry weight.

- Individual dose:

- Apply the Procedure outlined in Section 3 together with Equations (1) to (4) provided in the box above. As no activity concentrations are given for 226Ra, the values for 238U are those given in Tables 3.1 and 3.2 for U-238.

Thus,	Eind,crew=6.0´10−2mSv⋅kga⋅Bq´170Bqkg+4.6´10−2mmSv⋅kga⋅Bq´80Bqkg+3.4´10−3mSv⋅kga⋅Bq´140Bqkg14.4mSva−1

Eind,public=5´108kga−11´109kga−1´7.3´10−2mSv⋅kga⋅Bq´170Bqkg+5.0´10−2mSv⋅kga⋅Bq´80Bqkg

- Compare results with the individual dose criterion of 10 µSv a-1.

- The individual dose for the crew exceeds the individual dose criterion of 10 µSv a–1.

- Conclusion:

- The candidate material does not satisfy the individual dose criterion and cannot therefore be considered as de minimis under the London Convention on the basis of this simple assessment. There are two options for dealing with this situation:

- the material may be defined as not de minimis without further consideration, and therefore not suitable for disposal under the London Convention;

- a more detailed specific assessment may be performed, using Appendices I and II as a reference, to determine whether the material may be de minimis on the basis of a more detailed assessment.

annex iiI
INFORMATION ON CAUSES/SOURCES AND LABORATORY
RADIOMETRIC PROCEDURES

This Annex contains general information in Table III-I that may be useful to characterise the radionuclides likely to be present in the candidate material. Tables III-II and III-III list various facilities and industries which may generate or use radioactive materials and thus could indirectly affect the radionuclide composition of a candidate material. The radionuclides typically associated with each source are also indicated.

Examples of laboratory procedures for different sample types are provided in Table III-IV.

The amounts of radioactivity in waste materials used to define the sea disposal practice are to be compared with the IAEA exemption criteria, which are expressed as individual and collective doses in units of mSv a-1 and man/Sva-1, respectively. The activity concentrations should therefore be representative of the average composition of bulk material disposed over the course of one year. Particular caution should therefore be exercised in using activity concentrations based on values for small quantities of material or the results of analyses of spot samples of materials from batches or shipments. Appropriate steps should be taken by the national authority to ensure that reasonable estimates of average activity concentrations, over periods of a year, are used in conducting a specific assessment to determine if candidate materials are de minimis.

TABLE III-I. GENERAL INFORMATION USEFUL FOR THE SPECIFIC ASSESSMENT PROCESSES

- Unmodified Natural Materials - There is reason to believe that there is material known to be high in naturally occurring radioactive materials in the vicinity of the candidate material (e.g. monazite sands) - Anthropogenic Sources: Modified Natural Materials (This process is likely to increase the concentration of naturally occurring radioactive materials) - There are sites in the vicinity of the candidate material which have been involved in the processing or production of naturally occurring radioactive materials (e.g. radon, phosphorus compounds, vanadium compounds, refractory materials) or mining, milling or processing. - Uranium, thorium or radium compounds are used in manufacturing, research or testing in the vicinity of the candidate material, or these compounds are stored in the vicinity of the candidate material. - Coals or coal products are used in the vicinity of the candidate material, and combustion of the substance leaves ash or ash residue. - The processing of pipes from the oil and gas industries occurs in the vicinity of the candidate material. - Anthropogenic Sources: Artificial Radionuclides - There are facilities in the vicinity of the candidate material that are regulated for the manufacture, use, distribution of radioactive material, disposal, storage or incineration of radioactive waste (cases of leakage or release). - There is (or has been) any decontamination, maintenance or storage of radioactively contaminated ships, vessels, or platforms performed in the vicinity of the candidate material. - Additional Questions Regarding the candidate material - For calculations involving inhalation and/or dispersion: - What is the form of the candidate material? - What is the particle size distribution of the candidate material? - For calculations involving crew and public exposures: - To what extent is the candidate material contained during and following the disposal operation?

TABLE III-II. EXAMPLES OF NATURAL RADIONUCLIDES IN INDUSTRY

| Industry | Feedstock | Waste/by products |
| --- | --- | --- |
| Phosphate production | Phosphate rock: 1700 Bq kg-1 U-238 | Phosphogypsum solid: Ra-226 Liquid effluent : Po-210 |
| Metal production |  |  |
| Tin | Ores: tinstone, stannite ~ 1000 Bq kg-1 U-238; 300 Bq kg-1 Th-232 | Volatilisation of Po-210 Slags: 1000 Bq kg-1 U-238 300 Bq kg-1 Th-232 Lead/bismuth by product may also be active |
| Niobium | Ores: pyrochlore 10,000 Bq kg-1 U-238; 80,000 Bq kg-1 Th-232 | Slags containing U-238 and Th-232 |
| Rare earths | Monazite 10,000 Bq kg-1 Th-232 | Slags containing Th-232 (unless Th separated) |
| Thorium | Monazite | Product 100,000 Bq kg-1 Th-232 |
| Titanium | Ores: ilmenite, rutile U-238 and Th-232 70–9000 Bq kg-1 | Volatilisation of Po-210 |
| Refractory materials | Ores: zircon, baddelyite 10,000 Bq kg-1 U-238 and Th-232 | Volatilisation of Po-210 |
| Energy production |  |  |
| Coal combustion | Coal (typical): K-40, 50 Bq kg-1; U-238, 20 Bq kg-1; Th-232, 20 Bq kg-1. Some coals several times higher. One report of 15,000 Bq kg-1 U-238 in high uranium area | 0.5%–3% released to atmosphere, remainder in ash |
| Oil combustion | Oil | During extraction: Ra-226 in scale in pipework up to 4000 Bq kg-1 |
| Natural gas combustion | Radon up to 50 Bq l-1 | During extraction: Ra-226 in scale |
| Other industries |  |  |
| Uranium mining and milling | Ores: 0.2%–20% U308 | Tailings containing daughter radionuclide of U-238 plus up to 0.01% uranium Radon released depends on cover Abandoned tailings 3 Bq m-2 s-1 Ra-226 conc. 100,000 Bq kg-1 (1% ore) Radon also emitted from mines: 300 GBq t-1 ore mined |
| Precious materials – gold, etc |  | Tailings can contain U-238 500–2000 Bq kg-1 and/or Th-232 1000–2000 Bq kg-1 |

TABLE III-III. EXAMPLES OF ARTIFICIAL RADIONUCLIDES IN INDUSTRY

| Activity | Radionuclides |
| --- | --- |
| Nuclear industry | S-35, Ca-45, Cr-51, Mn-54, Fe-55, Fe-59, Co-57, Co-58, Co-60, Zn-65, Sr-89, Sr-90, Zr-95, Nb-95, Tc-99, Ru-103, Ru-106, Ag-110m, Sn-113, Sb-124, Sb-125, I-129, I-131, Cs-134, Cs-137, Ce-141, Ce-144 Pm-147, Np-237, Pu-238, Pu-239, Pu-241, Pu-242, Am-241, Cm-242, Cm-243 |
| Medical/clinical facilities1 | Na-22, S-35, Cl-36, Ca-45, Cr-51, Fe-59, Co-57, Co-58, Co-60, Se-75, Sr-85, Sr-89, I-125, I-131, Ir-192 |
| Consumer/industrial products2 | Pm-147, Ir-192, Tl-204, Th-232, Pu-238, Am-241 |

TABLE III-IV. LABORATORY RADIOMETRIC PROCEDURES

| Sample type | Example Radionuclides | System | Description | Application | Remarks |
| --- | --- | --- | --- | --- | --- |
| Sediment | 137Cs, 60Co, 226Ra (214Bi)a, 232Th (228Ac)a | Sodium iodide detector with multichannel analyzer (MCA) | Sodium iodide crystal, connected to a photomultiplier tube and MCA. | Laboratory gamma spectroscopy to determine the identity and concentration of gamma emitting radionuclides in a sample. | Analysis programs have difficulty if sample contains more than a few isotopes. |
|  |  | Germanium detector with multichannel analyzer (MCA) | Germanium semiconductor cooled in liquid | Laboratory gamma spectroscopy to determine the identity and concentration of gamma emitting radionuclides in a sample. | Especially powerful when more than one radionuclide is present in a sample. |
|  | Uranium isotopes; Plutonium isotopes; Thorium isotopes, other alpha emitters | Alpha spectroscopy | A system using silicon diode detectors for alpha energy identification and quantification. | Identifies and measures the activity of multiple alpha radionuclides in a thin extracted sample of soil, water, or air filters. | Sample requires radiochemical separation or other preparation before counting. |
|  | Beta emitters | Liquid scintillation counter (LSC) | Samples are mixed with LSC cocktail and the radiation emitted causes light pulses with proportional intensity. | Laboratory analysis of alpha or beta emitters, including spectrometry capabilities. | Highly selective for alpha or beta radiation by pulse shape discrimination. Requires LSC cocktail. |
| Water | 137Cs, 60Co, 226Ra (214Bi)a, 232Th (228Ac)a, 235U | Sodium iodide detector with multichannel analyzer (MCA) | See above. | See above. | See above. |
|  |  | Germanium detector with multichannel analyzer (MCA) | See above. | See above. | See above. |
|  | Uranium isotopes; Plutonium isotopes; Thorium isotopes, other alpha emitters | Alpha spectroscopy | See above | See above | See above |
|  |  | G.M. or Gas-flow proportional counter (lab) | Windowless (internal proportional) or window <0.1 mg/cm2, probe face area 10 to 20 cm2. May have a second or guard detector to reduce background and MDA. | Laboratory measurement of water, air, and swipe samples. | Requires counting gas. Windowless detectors can be contaminated. |
|  |  | Liquid scintillation counter (LSC) | See above | See above | See above |
|  | Beta emitters | Liquid scintillation counter (LSC) | See above. | See above. | See above. |

a Indicates that a member of the decay series is measured to determine activity level of the parent radionuclides of primary interest.

Contributors to Drafting and review

Amundsen, I.	Norwegian Radiation Protection Authority, Norway

Cabianca, T.	International Atomic Energy Agency

Clark, M.	Environmental Protection Agency, United States of America

Cunningham, J.	Radiological Protection Institute of Ireland, Ireland

Engler, R.	Army Corps of Engineers, United States of America

Linsley, G.	International Atomic Energy Agency

Monte, L.	ENEA, Italy

Nauke, M.	International Maritime Organization, United Kingdom

Nielsen, S.P.	Risø National Laboratory, Denmark

Robinson, C.	International Atomic Energy Agency

Smith, B.	Centre for Environment, Fisheries and Aquatic Science, United Kingdom

Steevens, J.	Army Corps of Engineers, United States of America

Thierfeldt, S.	Brenk Systemplanung, Germany

Torres, C.	International Atomic Energy Agency

Vivian, C.	Centre for Environment, Fisheries and Aquatic Science, United Kingdom

Vogt, C.	Environmental Protection Agency, United States of America

Technical Committee Meetings

Vienna, Austria: 29 May – 2 June 2000

Consultants Meetings

Vienna, Austria: 5–9 July 1999

Vienna, Austria: 13–17 December 1999

Vienna, Austria: 20–24 March 2000

Vienna, Austria: 29 May – 2 June 2000

Includes radionuclides derived from authorised discharges.

For example, ICRP 60 [4] includes the following statement: “The Commission believes that the standard of environmental control needed to protect man to the degree currently thought desirable will ensure that other species are not put at risk. Occasionally, individual members of non-human species might be harmed, but not to the extent of endangering whole species or creating imbalance between species.”

These include, for example, Agenda 21 of the Rio Declaration on Environment and Development 1992, the London Convention and the OSPAR Convention, 1998.

If not otherwise specified, the term ‘dose’ is used in this Report to indicate the committed effective dose from intakes (ingestion or inhalation) and the effective dose for external exposure as defined in the International Basic Safety Standards [7].

The term ‘exemption for the purposes of the London Convention’ is taken to mean ‘de minimis’, that is, it includes both the radiological concepts of exclusion and exemption.

The value for members of the public given in the table was calculated for infants as this is more restrictive.

The value for members of the public given in the table was calculated for infants as this is more restrictive.

Values for U-238 should be used when activity concentrations of Ra-226 are available. If these activity concentrations are not available, values for U-238s should be used instead.

Values for U-238 should be used when activity concentrations of Ra-226 are available. If these activity concentrations are not available, values for U-238s should be used instead.

All of the following pathways are included in the screening assessment.

Wastes or other matter that may be considered for dumping (1996 Protocol to the London Convention, Annex I).

External radiation to the crew is work-related and to the public is related to recreation.

Inhalation by the crew is work-related and by the public is related to recreation.

Ingestion by the crew is work-related and by the public is related to seafood consumption.

ü ü ü = Likely = ü ü = Unlikely; ü = Very unlikely

Radiation Protection Authorities of Contracting Parties may use different generic values.

Unless explicitly provided in the table (value in brackets next to the radionuclide), weighting factors are equal to 1.

For Np-237 and Pu-241, only a fraction of the entire decay series has been included because the remaining progeny in the decay chain cannot build up to give any siginificant activity or dose contribution.

No subchains have been provided for Th-232 as the chain reaches equilibrium within a few decades

No subchains have been provided for U-235, as the activity of U-235 is only a few percent to the activity of U-238 and therefore, the contribution of all progeny can always be included;

The entire decay chain U-238sec should be used in those cases where equilibrium between all radionuclides is a reasonable assumption, like in geological material, sediments etc.

This subchain of U-238 is necessary for calculations in cases where there is disequilibrium between uranium and radium isotopes, as might be the case when chemically separated uranium is a contaminant.

1	These Guidelines were adopted in 1999 at the Twenty-first Consultative Meeting of Contracting Parties to the London Convention (LC 21/13, paragraphs 6.1 - 6.14, annex 6).

2	This guidance is meant specifically for implementation of the de minimis aspects of Annex I to the London Convention, and may have to be adapted for implementing the 1996 Protocol to the London Convention upon its entry into force.

3	The IAEA has been tasked by the Twentieth Consultative Meeting to provide more detailed guidance to national authorities for conducting specific assessments (LC 20/14, paragraph 7.16).

4	The text in this evaluation procedure refers to the national radiation protection authority and the national permitting authority. It is recognized that these authorities could be the same agency in some countries, could be called by other titles, or could encompass more than two agencies.

1	Biomedical research (Cr-51: blood cell survival; Se-75: protein studies; Sr-85: bone formation studies), clinical diagnosis and measurements (Co-57: diagnosis of pernicious anemia; I-125: diagnosis of thyroid disorders), cancer treatment (Sr-89: reduction of pain from bone cancer; I-131: treatment of thyroid disorders), instrument sterilization.

2	Luminous devices, thickness gauges, smoke detectors, gas mantles, welding electrodes, pipe testing devices, power sources for satellite.